MEKPCO FLARE NETWORK DESIGN

ASPEN FLARENET

Content

- Description

- Design Procedure

  - Select proper Orientation

  - Select and Size proper Inlet Device, Inlet and Outlet ID

  - Calculate Vessel Diameter

  - Calculate Vessel Height

  - Select and Size Manholes, Vent, Drain, Vortex Breaker

  - Select a well-designed mist eliminator pad

Flare System Design Procedure

In order to design a flare package, the following procedure is taken:

- Determination of worst-case scenarios

- Flare network line sizing using Aspen flarenet and API-527

- Flare K.O.D sizing using API-527 excel sheet

- Stack height and diameter calculation by API-527

System Description

The overall Flare system consists of three parts:

1. The first part is made of flare network and K.O.D inside MEKPCO plant. The second part is

made of flare main pipes between the tie-in point of the plant and flare area at the northeast.

The third part consists of flare stack package which will be located on the north mountain. The

stack will be located at X:652388, Y:3051685, Z:135.

2. The flare main header approximate length is considered 2715 for hydraulic analysis

implementation, including the flare piping length 305m between the starting point and the tie-in

point in methanol plant and the flare piping length 2410 between the tie-in point and the actual

flare site.

3. For the flare piping routing, one stress expansion loop is set at about 60 m distance, about

27 stress expansion loops between the tie-in point and the flare stack.

4. For the flare piping routing, the elevation changes between the tie-in location of

methanol plant and the actual flare site is about 111m.

5. The backpressure at the flare stack is 1.4 bara.

The Whole Plot

In Site Flare Header

Criteria and Design Basis

The sizing of relief discharge piping can usually be simplified by starting at the system outlet,

where the pressure is known, and working back through the system to verify acceptable

backpressure at each PRD Calculations is performed in a stepwise manner for each pipe

segment of constant diameter.

Several methods for calculating the size of discharge piping have been developed using

isothermal or adiabatic flow equations. Actual flow conditions in relief systems are normally

somewhere between isothermal and adiabatic conditions. For most cases, the slightly more

conservative isothermal equations are recommended.

- For both header and tailpipes, the Mach number of both 0.1 and 0.5 are taken into account.

- The maximum ρν2 for the whole piping should be 150000 kg/m/s2

- The sound/ noise level should not exceed the recommended value of 100 DBA

Determination of worst-case scenario

All possible sources of relief load are summarized below and the worst-case scenario is

detected.

Flare network line sizing using Aspen flarenet and API-527

Procedure: The discharge load of each release point, the backpressure of flare stack, the

diameter and length of each pipe segment is given to seek the Mach number and

backpressure alongside each pipe segment.

1st Step: Create the component and set the unit

2nd Step: Checking “Calculation Setting”

Note: There is no need to change PSV sizing, Solver, Initialization and AIV tabs in calculation

setting.

3rd Step: Create the layout

4th Step: Definition of Scenarios

In order to define a scenario, at first “clone” a new scenario from default scenario and then

complete the Scenario. Also, in “sources” tab, do not select the PSVs included in the scenario.

5th Step: Act accordingly and Run the software

Results

Solution

In order to resolve the warnings, pipe diameters (all 12’) are increased till the constrains are

met. The Starting point is from last pipe segment. When the pipe ID is increased to 54’ then the

Mach number for last segment is decreased to 0.46. it was observed that only last pipe has

experience Mach number violation.

This could be justified by the fact that the pressure in other pipe is too high that the volumetric

flow has reduced dramatically in a way that Mach number is not a problem for these pipes.

Interesting!!

Then other pipe segment diameter were increased and when other pipe ID, one by one,

reached 46’, the whole header and tail pipes Mach number met the constraints. By the way for

smaller diameters like 42’ the tests were carried out but the pipes in the middle of header could

not bear and Mach number criterion was not met.

Also, tail pipe ID were increased to 20’ but the backpressure on PSV-2356-60 was higher than

the set value, therefore, the PSV type were changed to balanced type and their compatibility to

system backpressure was increased and as a result, the warning disappeared. The final

diameter for tail pipes were 12’ and smaller diameter were tested but the Mach number criterion

failed. So, 12’ was selected.

Also, case B and C were tested and for PV-2481, the pipe ID of 28’ was suitable. For PV-

2536B the pipe ID of 8’ was found suitable.

All above calculation are based on Mach number of max 0.5. It is interesting to notice that in

Marjan Project the main header before K.O.D is 48’ and the header after K.O.D to flare site is

expanded to 64’. It seems that in Marjan Project the max Mach number of 0.5 has been

considered as basis of design.

In MEKPCO Project, the max Mach number of 0.1 has been regarded as design basis and as a

result, the main header before K.O.D is 56’ and the header after K.O.D to flare site is expanded

to 64’.Also ,in next page the comparison between IMPLANT and ASPEN FLARENET results for

Mach number of 0.1 is shown.

Case A

| PSV-2354 | PSV-2355 | PSV-2356 | PSV-2357 | PSV-2358 | PSV-2359 | P-52-in | P-52-out | P-57-in Mach | P-57-ou Mach |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.836 | 2.837 | 2.838 | 2.845 | 2.849 | 2.85 | 2.474 | 2.427 | 0.11 | 0.129 |
| 2.02 | 2.05 | 2.12 | 2.17 | 2.17 | 2.13 | 1.55 | 1.48 | 0.18 | 0.33 |

Case B

| PV-2481 | USV-2482 | PV-2536 | P-52-in | P-52-out | P-57-in Mach | P-57-out Mach |
| --- | --- | --- | --- | --- | --- | --- |
| 1.37 | 1.36 | 1.34 | 1.22 | 1.20 | 0.08 | 0.09 |
| 0.87 | 0.99 | 0.78 | 0.67 | 0.65 | 0.11 | 0.14 |

Discussion

It sounds that TCC has set Mach number of 0.1 as the criterion in order to have noise level

within reasonable limits. Furthermore, Implant and Aspen Flarenet results are compared which

shows good Compatibility.

Also, the deviation might stem from different composition because in absence of exact

composition, MW Of each source were given to Aspen Flarenet and aspen flarenet estimates

the composition.

Flare K.O.D Sizing

1.Knockout drums are typically located on the main flare line upstream of the flare stack or any

liquid seal. If there are particular pieces of equipment or process units within a plant that release

large amounts of liquid to the flare header, it is desirable to have knockout drums inside the

battery limits to collect these liquids. This reduces the sizing requirements for the main flare

knockout drum, as well as facilitates product recovery.

2. The economics of drum design can influence the choice between a horizontal and a vertical

drum. If a large liquid storage capacity is desired and the vapor flow is high, a horizontal drum is

often more economical. Also, the pressure drops across horizontal drums is generally the lowest

of all the designs. Vertical knockout drums are typically used if the liquid load is low or limited

plot space is available. They are well suited for incorporating into the base of the flare stack.

3. Among disparate configurations proposed in API-527, a combination of a vertical drum in the

base of the flare stack and a horizontal drum upstream to remove the bulk of the liquid entrained

in the vapor has been selected for MEKPCO. Horizontal drum with the vapor entering one end

of the vessel and exiting at the top of the opposite end (no internal baffling);

4.A split-entry or split-exit configuration can be used to reduce the drum diameter (but increase

the length) for large flow rates and should be considered if the vessel diameter exceeds 3.66 m

(12 ft). Careful consideration should be given to the hydraulics of split-entry configurations to

ensure the flow is indeed split in the desired proportion. Inlet nozzles should include means

such as baffles or long sweep elbows to prevent re-entrainment of liquid. Long sweep elbows

are typically used up to DN 300 (NPS 12) inlet diameter. Baffles are typically used for larger

inlet diameters. Neither of these rules has been applied for MEKPCO application.

5. In general, vapor outlet nozzles should not be fitted with any devices (e.g. deflection plates,

baffles, demister pads, vane packs, etc.), because of the potential for such devices to fail or

plug and obstruct the outlet. Such devices should be used only if the drum is equipped with an

alternate outlet nozzle sized for the drum’s design vapor flow rate and fitted with a rupture disk

(or pin-actuated device) whose burst pressure is selected both to protect the drum against

overpressure and to permit proper operation of the drum and relief system in the event the

normal vapor outlet becomes obstructed.

6. Liquid droplets 300 um and larger may drop out of the gas stream at less than 2 m/s. If liquids

are not drained from the system, flare flows with gas velocities exceeding about 3 m/s or 4 m/s

can entrain liquid droplets up to 1000 μm in size. Liquid droplets exceeding 1000 μm can readily

lead to burning rain regardless of flare type.

7. Among different flare burner types alluded to in API-527 steam and air-assisted flare burner

has been selected for MEKPCO project. For this design, Flare gases containing less than 1 %

by mass of liquids up to a liquid droplet size of 600 μm can be handled smokeless and without

burning rain. Some air assisted burners with small ports and operated at significant pressures

can handle larger amounts, and with larger droplet size, without smoke.

8. The liquid holdup capacity of a flare knockout drum is based on consideration of the amount

of liquid that can be released during an emergency situation without exceeding the maximum

level for the intended degree of liquid disengagement. The holdup times vary between users,

but the basic requirement is to provide sufficient volume for a 20 min to 30 min emergency

release. Longer holdup times might be required if it takes longer to stop the flow. This holdup

should also consider any liquid that can have previously accumulated within the drum that was

not pumped out. If there has been a liquid discharge to the knockout drum whereby the liquid

level exceeds the maximum level required for adequate vapor-liquid separation, then liquid shall

be removed to reduce the level back below this maximum level.

9. It is important to realize as part of the sizing considerations that the maximum vapor release

case might not necessarily coincide with the maximum liquid. Therefore, the knockout drum size

should be determined through consideration of both the maximum vapor release case as well as

the release case with the maximum amount of liquid. If no valid liquid case exists and the vapor

is either condensable or has a condensable component, then the design liquid case should be a

minimum of 2 wt. % of the maximum gas rate to the flare knockout drum.

1st Step: Nozzle Sizing

The criterion for inlet nozzle sizing is velocity limitation which should be 7-13 m/s.

The criterion for gas outlet nozzle sizing is velocity limitation which should be 15-30 m/s.

The criterion for inlet nozzle sizing is velocity limitation which should be 2-4 m/s.

| Parameter | Inlet Nozzle | Gas Outlet Nozzle | Water Outlet Nozzle |
| --- | --- | --- | --- |
| Mass | 540000 kg/h | 510000 kg/h | 30000 kg/h |
| Density |  | 6.82 kg/m3 | 988 kg/m3 |
| Qv | 20.25m3/s | 20.24 m3/s | 0.01 m3/s |
| ID-TCC | 64’ | 72’ | 4’ |
| Velocity | 9.76 m/s | 7.7 m/s | 1.04 m/s |
| My ID | 64’ | 64’ | 3’ |
| Velocity | 9.76 m/s | 9.76 m/s | 1.85 m/s |

Note that according to API-521, when there is no criterion stipulating the exact amount of water

which should be separated a minimum of 2% by weight of total vapor mass could be selected as

basis for calc.

But TCC has assumed 30000 m3/h as the rated capacity of K.O.D pump which is Another

matter is that the vendor has selected 72’ as outlet nozzle ID which is a bit bigger than what the

criterion stipulates. Perhaps the vendor was more concerned with droplet sizing so that the

higher the outlet nozzle ID, the lower particle sizing. Probably maximum 300 um has been taken

for Design Basis.

2nd Step: K.O.D Diameter and Length Sizing

1. Assume Diameter and consider L= 2.5 or 3 D

2. Calculate AL and AT according to the following equation:

AL = Q.t/ (L.N)     AT = П(D^2)/4      t = Hold-up time

3. Calculate AL/AT and calculate hL/D from the following equation:

4. Calculate hV = D. ( 1 – hL/D)

| Parameters | Values |
| --- | --- |
| Vapor mass kg/h | 510000 |
| Liquid mass kg/h | 30000 |
| ρv Kg/m3 | 6.8 |
| ρl Kg/m3 | 990 |
| Dp um | 300 |
| Hold-up time minute | 30 |
| L/D | 3 |
| D meter | 3.5 |
| L meter | 10.5 |

5.Calculate uc by following equation:

6. Calculate φ = hv/uc

7. Calculate AV = AT-AL

8. Calculate Uv = Qv / (Av.N)

9. Calculate Lreq = Uv. φ .N

| Parameters | Value |
| --- | --- |
| AL | 1.44 |
| AT | 9.62 |
| AL/AT | 0.15 |
| hL/D | 0.21 |
| ERROR | Goal Seek Function |
| hV/D | 0.79 |
| hV | 2.77 |
| Uc | 0.75 |
| φ | 3.7 |
| AV |  |

| Parameters | Value |
| --- | --- |
| Uv | 2.55 |
| Lmin m | 9.42 |
| Inlet ID inch | 64 |
| Outlet ID inch | 72 |
| LT | 14.6 |

| Parameters | Value |
| --- | --- |
| AL | 6.2 |
| AT | 9.62 |
| AL/AT | 0.64 |
| hL/D | 0.61 |
| ERROR | Goal Seek Function |
| hV/D | 0.39 |
| hV | 1.35 |

If initial volume is not zero and is about 50 m3

| Parameters | Value |
| --- | --- |
| Uc | 0.75 |
| φ | 1.8 |
| AV | 3.42 |
| Uv | 6.1 |
| Lmin m | 10.98 |
| Inlet ID inch | 64 |
| Outlet ID inch | 72 |
| L | 10.5 |

10.Verification:  If L Req is greater than L, increase the drum diameter and repeat the L Req

calculations again until L Req is less than L. Also, hl/D should be max 0.5.

Since L Req is less than L then, we ought to increase K.O.D ID to 4 meter and repeat the

procedure.

So for doing this, we use Goal seek function in Excel to help us reach exact point of hL/D.

| Parameters | Value |
| --- | --- |
| D | 4 |
| L/D | 3 |
| L | 12 |
| Uc | 0.75 |

| Parameters | Value |
| --- | --- |
| AL | 5.43 |
| AT | 12.57 |
| AL/AT | 0.43 |
| hL/D | 0.44 |
| ERROR | Goal Seek Function |
| hV/D | 0.55 |
| hV | 2.21 |

| Parameters | Value |
| --- | --- |
| φ | 2.95 |
| AV | 7.14 |
| Uv | 2.92 |
| Lmin m | 8.62 |
| Inlet ID inch | 64 |
| Outlet ID inch | 72 |

TCC has ordered a K.O.D with the following sizing which is oversized in my opinion but

appears to be good in handling such amount of gas and condensate.

| Parameters | TCC | API-521 | Sabalan |
| --- | --- | --- | --- |
| Diameter | 5.8 m | 4 m | 5m |
| Length | 18 m | 14 m | 10m |

Flaresim Design

Flare Thermal Radiation Assessment

Based on

API-521 and Schlumberger Flaresim

Calculation of Flare Stack Diameter

In order to calculate Flare Diameter, the following equation between Mach number and

Diameter is used.

According to TCC calculation a flare stack with diameter of 1.6 m and a height of 44.45 m

Here according to API-521 , the calculated diameter is about 2.46 m when the temperature is

273 K and when the temperature is 633 K for blocked outlet scenario then the calculated

diameter is 3.04 m.

| Parameters | Value |
| --- | --- |
| T | 273.15 K |
| Z | 1.33 |
| M | 13.44 |
| P2 | 141 kpa |
| Mach | 0.1 |
| DD | 2.46 |

Calculation of Flame Length

1.Calculate liberated heat by following formula and then use next page Figure to determine

Flame Length.

Q = Mass (kg/h) . LHV (kJ/kg) . (1h/3600s)

Q = 712342 Nm3/h . 7964 / 3600 = 1.57 × 10^6 kW

According to the next-page diagram the estimated Flame Length is round about 70 m.

Calculation of Flame Distortion Caused by Wind Velocity

- Calculate vapor volume flow rate

- Divide wind velocity to flare tip velocity

- Use next-page diagram to calculate ∆X and ∆Y.

- Use distance equation and plot to estimate flare stack height.

1st Step

T = 273K

qv = (51000/3600)×22.413.4× (273/273) = 236.8 m3/s

T = 633K

qv = (51000/3600)×22.413.4× (633/273) = 549 m3/s

2nd Step

In order to perform this step, at first, we need to calculate flare tip exit velocity according to

following formula

T=273K

uj = 49.6 m/s

T=633K

uj = 75 m/s

So now wind velocity to tip velocity is divided by taking into account 16 m/s as local wind

velocity

T=273K

u/uj = 0.32

T=633K

u/uj = 0.21

T=633K

According to next-page diagram, ∆Y/L = 0.3 therefore ∆Y = 21m

According to next-page diagram, ∆X/L = 0.9 therefore ∆X = 63m

T=273K

According to next-page diagram, ∆Y/L = 0.23 therefore ∆Y = 16m

According to next-page diagram, ∆X/L = 0.93 therefore ∆X = 65m

A common approach to determining the flame radiation to a point of interest is to consider the

flame to have a single radiant epicenter and to use the following empirical equation by Hajek

and Ludwig. Equation (F.1) may be used for both subsonic and sonic flares, provided the

correct F-factor is used.

| Parameters | Values |
| --- | --- |
| τ | 1 |
| F | 0.2 |
| Q | 1.57 ×10^6 |
| K | 3.13 |
| D | 89.5 m |

| Parameters | Values |
| --- | --- |
| τ | 0.8 |
| F | 0.15 |
| Q | 1.57 ×10^6 |
| K | 3.13 |
| D | 70 m |

F-factor

T=233K

In order to calculate flare height, the following equation is used.

| Parameters | Values |
| --- | --- |
| r | 35 |
| ∆X | 65 |
| r’ | 2.5 |

| Parameters | Values | Values |
| --- | --- | --- |
| D | 89.5 | 70 |
| r’ | 2.5 | 2.5 |
| h’ | 89.4 | 69.9 |

| Parameters | Values | Values |
| --- | --- | --- |
| h’ | 89.4 | 69.9 |
| ∆Y | 16 | 16 |
| h | 81.4 m | 62 m |

T=633K

In order to calculate flare height, the following equation is used.

| Parameters | Values |
| --- | --- |
| r | 35 |
| ∆X | 63 |
| r’ | 3.5 |

| Parameters | Values | Values |
| --- | --- | --- |
| D | 89.5 | 70 |
| r’ | 3.5 | 3.5 |
| h’ | 89.4 | 69.9 |

| Parameters | Values | Values |
| --- | --- | --- |
| h’ | 89.4 | 69.9 |
| ∆Y | 21 | 21 |
| h | 78.4 m | 59 m |

An average Flare Stack Height of 70 m is accepted for preliminary design which should be

confirmed by Flaresim.

In MEKPCO project there is one set of elevated flare in Flare system which is supported by

derrick. The total height of flare is 70 m (Flare stack + K.O.D =60m and Molecular Seal and

Flare tip each 5 meter)

1st Step: Click on “Fluid TAB” and “Add” a Fluid Table

2nd Step: Click on “Environment TAB” and “Add” a Environment Table

Thermal Radiation and Noise Criteria

According to API-521, the followings are important to notice when designing a flare stack.

Recommended Design Thermal Radiation for Personnel.

The incident solar radiation for the site. Typical values for different geographical locations are

given in the following table.

The decision on whether to include solar radiation when designing flare systems is one for the

user.

API 521 recommends that this be considered on a case-by-case basis. Some consider it more

realistic to exclude solar radiation in calculations.

In deciding whether to include solar radiation consideration should be given to the frequency

and duration of the flaring event, the probability of personnel being present in the exposed

location, the possibility of sun and flare radiation impinging from the same general direction, the

likelihood of protective clothing being worn and the ease or difficulty of escape from the exposed

location.

Including solar radiation leads to more conservative designs and its impact can be significant if

the flare system design is controlled by low total radiation limits at longer distances from the

flare.

Determining whether solar radiation has been included or excluded is important when

comparing flare system designs.

In MEKPCO Project, solar radiation has been included.

Noise

The background noise is used as a reference noise level to which the noise from the flare

system is added.

In MEKPCO Project, a typical value of 60 dB has been selected for background noise.

Stability Class

PasquillA through PasquillF

This defines the atmospheric stability class to be used to characterize the atmospheric

turbulence for both Gaussian and jet dispersion calculations.

Flaresim uses the widely used Pasquil stability class designation from A to F where A is the

most turbulent or most unstable atmosphere and F the least turbulent or most stable.

Here as typical, PasquillD is selected.

10-meter high is reference height at which the wind speed is specified. This will be used

together with the atmosphere and terrain characterization information to calculate the wind

speed at a given height when required.

3rd Step: Click on “Stack TAB” and “Add” a Stack Table

At first estimation, a stack length of 65 m is selected. Since the stack is vertical 90 degrees for

Horizontal angle is selected.

Notice that Noise A means Noise weighted for human ear sensitivity

4th Step: Click on “Tips TAB” and “Add” a Tip Table

For gases, either the pipe or sonic tip types may be selected. Pipe flares are the simplest type

of tip and may be specified for both high- and low-pressure gases. If the pressure available is

greater than 2 bar (30 psi) at the tip then a sonic tip can be utilized. Sonic flare tips have the

advantage of low flame emissivities due to more efficient combustion of the flare gas. For lower

pressures a pipe flare is generally used possibly with steam or air Assist fluid.

Where a combined HP/LP tips is selected the HP tip is assumed to be a sonic tip and the LP a

sub-sonic one. The flow ratio of HP to LP fluids should be 3 or greater.

For liquids a Welltest tip type should be selected.

In MEKPCO Project, Pipe is selected.

Seal Type

There are two basic types of seal, Fluidic or Molecular. Seals are only appropriate for pipe and

sonic flare tips. If the tip type is set to Welltest the seal type will be set to None automatically.

The figure below shows the general design concept for the fluidic seal.

The type selection is a function of the opening as defined below

- Fluidic1: 50% of total area

- Fluidic2: 40% of total area

- Fluidic3: 35% of total area

The figure below shows the general design concept for the molecular seal.

The type selection is a function of the diameter as defined below:

Molec.1: Traditional design. Maximum diameter is 1.7 times the tip diameter. The pressure drop

correlation is based on a design which gives a body length of 5.5m (18ft) regardless of the tip

diameter.

Molec.2: Low pressure drop design. Maximum diameter is 2 times the tip diameter. The

pressure drop correlation is based on a design which gives a body length which is a function of

the tip diameter.

The fluidic seal has a number of advantages over the traditional molecular seal:-

- Lower purge gas requirements and consequent operating costs.

- The seal still operates with a high efficiency even if rain water or chunks of refractory

material drops into the baffles. In fact, the water is quickly dissipated because the fluidic

seal is located at a high temperature section of the flare stack.

- Lower cost due to the simple construction and light weight. A 48" fluidic seal will typically

weigh less than half the weight of a 6" molecular seal.

In MEKPCO Project, Molecular 1 is selected.

Seal Diameter Basis

This entry defines whether the seal calculations are based on the tip diameter or stack diameter.

effectively this specifies whether the seal is located within the tip or the stack.

Here Stack has been selected.

F Factor

The fraction of the total net heat release from the flame which is radiated.

- Natural gas: Correlation based on tip exit velocity assuming a natural gas fluid of

molecular weight 19.

- Tan: Correlation based on mole weight

- Kent: Correlation based on mole weight

- High Efficiency: Proprietary correlation between tip type, exit velocity, fluid molecular

weight and degree of hydrocarbon saturation. Formally known as the Flaresim method in

versions prior to 1.2.

- Cook: Correlation based on exit velocity.

- Generic Pipe: Correlation based on refitting Kent, Tan, Natural gas and Cook methods

across a range of exit velocities and molecular weights.

- Modified. Chamberlain: Correlation based on mole weight and exit velocity.

Which one to select?

Where flare vendor data is available it should be used in preference to a correlation. In the

absence of vendor data, the Generic Pipe method is recommended for a conservative design.

For clean burning, smokeless flares from well-designed flare tips in good condition the High

Efficiency method can be used.

In practice this means flares burning paraffinic hydrocarbons of low molecular weight fluid (<60)

at reasonable exit velocities (> 0.2 Mach). For fluids other than paraffinic hydrocarbons vendor

advice should be sought. In the absence of advice, user specified F Factors of 0.3 to 0.4 are

generally reasonable.

If the Fraction Heat Radiated Method is set to User Specified then the required value for the

fraction of heat radiated must be entered here. Otherwise, the calculated result for the selected

calculation method will be displayed after the model has been run.

Here Generic Pipe is selected.

Flame Length Method

- API - Flame length is calculated from heat released according to equation presented in

API 521.

- FLARESIM - Flame length is calculated from heat released using following equation.

Where

L is flame length in m

Q is heat release in J/s
N is number of tips
The constants I1 and I2 take the following values for different tip types

- Brzustowski - Flame length is calculated from flammability limits using Brzustowski &

Sommer method

Here Brzustowski is selected by the Flaresim.

The physical length of the burner tip. The value is used in calculating the true gas exit point for

flame Length calculations and gas dispersion calculations.

For Exit Diameter calculation, click “Size Me” and enter 0.11 as Design Mach number in order to

allow the software to calculate proper exit diameter.

Noise input

Selects the noise calculation method to be used.

1. Acoustic Efficiency method

2. Reference Curve method

The Reference Curve method is based on a reference spectrum of noise at a known heat

release.

Select Noise Curve

1.Standard

2.Low Noise

The Standard and Low Noise reference data options are

proprietary data supplied by a flare system vendor.

5th Step: Click on “Calculation Option TAB” and “Add” a Table

Select the method to be used to model the thermal radiation from the flame.

- The Flaresim API and Strict API methods model the single point source method of Hajek

and Ludwig given in API RP-521. The difference between the methods is in the method

of calculating the flame shape before finding the center point to act as the source. The

Flaresim API method uses the vector-based flame shape method and allows multiple

flame elements to be used to model the shape more accurately even though a single,

center point will be used as the source. The Strict API method uses the graphical

method presented in API 521 through a curve fit to the data presented there. The API

method in DOS versions of Flaresim and Flaresim for Windows versions prior to version

2.0 was the Flaresim API method. Either API method may be generally applied to most

flare systems.

- The Point source method is a multiple point extension of the API method in which the

flame is assumed to be completely transparent such that radiation from one point does

not either interfere with or occlude another. The flame is divided into a series of smaller

point source elements whose contributions are summed to derive the total radiation from

the flame. In practice this method generally gives more realistic and less conservative

values than the API method. It does however tend to over predict thermal radiation in the

near field.

- The Diffuse source method assumes that the flame is completely opaque such that

radiation is emitted entirely from the surface of the flame envelope. This method tends to

under predict the thermal radiation in the near field.

- The Mixed source method is an empirical combination of both the Point and Diffuse

source methods. This has been found to give more realistic results in both the near and

far fields.

- The Brzustowski method is a single point method in which the flame center is

determined from jet dispersion theory. The method as described in API RP-521 is

subject to a number of limitations in its implementation in Flaresim: -

- Only vertical tips may be modelled.

- Air assisted flares may not be modelled.

- Liquid burners may not be modelled.

- The M.Point Brz method is a Flaresim extension to the standard Brzustowski method to

allow the number of flame elements and the element position to be specified by the user.

In versions of Flaresim prior to 1.2 these options could be set for the Brzustowski

method. In Flaresim 1.2 the Brzustowski method is forced to be a single flame element

with fixed element position. Old cases that specify the Brzustowski method will be

updated automatically to M.Point Brz if they have more than one flame element or the

element position is not 50%.

- The Chamberlain method, also known in the industry as the Shell Thornton method is

based on a modelling the flame as a conical frustum radiating from its surface with a

uniform emissive power. The method was developed to provide more accurate

predictions of flame shape and radiation in the near field.

Number of flame element

The number of elements that the flame is divided into for calculation of flame shape and

the sources for the Point, Diffuse and Mixed methods. Larger values will generally give

more realistic values for the thermal radiation at the expense of calculation time.

1 Element is set for this case.

Element Position

The element position indicates the source point within a flame element that is used for

calculations. Typically this is 50% i.e. the middle of the flame element is taken to be the

point source. 0% indicates the source is the start of the element, 100% is the end.

50% is set in this project.

Noise Method

Selects the method to be used for the noise calculations. The API method taken from

RP521 is a simple single value method and considers jet noise only. The Spectrum

method uses multiple frequency values and includes combustion noise. Generally, the

Spectrum method is recommended.

Spectrum is selected.

Windchill

When set an empirical Windchill correlation is used to correct the incident thermal radiation at

any receptor point by taking into account the heat losses due to passage of wind over the point.

Use of this option will generally be a matter of individual judgement or your company standards.

It is recommended that you do not use this option if you are interested in the surface

temperature calculations. Note that effective of wind on convective heat transfer in the surface

temperature calculations is independent of the setting of this option.

Atm. Noise Attenuation

When set a correction will be applied to the noise calculations to allow for the attenuation in

noise due to atmospheric absorption. This option should normally be set

Jet Dispersion

Selecting this enables the jet dispersion calculations and will calculate concentrations of flare

fluid at receptor points and for receptor grids under flame out conditions

Gaussian Dispersion

Selecting this enables the calculations for all Gaussian Dispersion objects defined in the model

Run Dynamics

Selecting this enables dynamics calculations to be run to evaluate the change in radiation and

other results with time as the flare flow varies with time. The variation in flare flow with time must

be defined on the Tip Dynamics View and the results variation with time can be viewed on the

Receptor.

Point Dynamics View

The dynamics calculations are run in addition to the defined base case. Receptor Grid objects

and Dispersion objects are not included int the dynamic calculations

Buoyancy

For all methods except the Brzustowski and Chamberlain methods, the flame shape is

calculated by resolving the velocity vectors in three dimensions. The main components are the

tip exit velocity and the wind velocity. There is however an additional velocity component which

is due to the density differences between the hot combustion gases and the surrounding air.

This is referred to as the flame buoyancy term.

Buoyancy - Pipe

The flame buoyancy which should be used for Pipe flares. A value of 3.0 m/s is recommended

unless specific vendor information suggests otherwise.

Buoyancy - Sonic

The flame buoyancy to be used for Sonic flare tips. A value of 4.6 m/s is suggested unless

specific vendor information suggests otherwise.

Buoyancy - Welltest

The flame buoyancy to be used for Liquid flare tips. A value of 0.03 m/s is suggested unless

specific vendor information suggests otherwise.

The recommended buoyancy values are based on empirical information supplied by a flare

vendor. The wide range of allowed values is intended to provide flexibility for users with specific

information and the validity of any values entered is the responsibility of the user.

6th Step: Click on “Receptor point TAB” and “Add” a Point

Results/Tip

Results/Point1

7th Step: Click on “Receptor Grid TAB” and “Add” a Grid1

Results/Grids

7th Step: Click on “Receptor Grid TAB” and “Add” a Grid2

Results/Grids

A Comparison Study

1.In Calculation Option TAB, Brzustowski method was changed to Flaresim API.

2.In “Options” just Atm. Noise Attenuation is enabled.

Results

| Parameter | Flaresim API | Brzustowski |
| --- | --- | --- |
| Stack Height | 68.4 | 70.3 |

Tip Result Comparison

Flaresim API

Point Result Comparison

Flaresim API

Notes and Discussion

- The radius of public , exposure limit ( 1.58 kW/m2 ) is 172m in Easting Direction.

- The maximum radiation value on ground is 3.135 kW/m2.

- If exposure limit for all direction is set to be 1.58 kW/m2 then the flare stack height

should be 152m!

- Vendor has selected a fixed height and no constraint in Point Tab but here we allowed

the software to size the stack height by setting 3.13 kW/m2 as sizing constraint in Point

Tab.

Flare Burner / Tip

In MEKPCO project, a steam-assisted flare tip is used to not only help the combustion be

flameless but also cool down the equipment.

The quantity of steam required for smokeless burning is a function of the gas composition, the

flare burner size and design, the steam-injector design and operating pressure, and the

environmental conditions. While steam assist enhances the combustion of relief gases that

smoke, it adversely affects the combustion of relief gases with a high level of inerts. Relief

gases with a high level of inerts, when flared from a steam-assisted flare, can require a greater

calorific value to sustain the required flame stability and hydrocarbon-destruction efficiency.

Flare tip length is normally between 3-5 meter. In MEKPCO project, pipe flare type is used due

to low Mach number.

Steam is often injected into the relief gas discharge at the top of a flare burner. Typically, a

steam ring that has a number of injection nozzles or slots is employed. The design and location

of injector nozzles varies as different flare manufacturers each have their own proprietary

design.

The upper steam injection functions to inspirate air and to force the air mixture into the relief gas

discharging from the flare burner. The steam-injection pattern is intended to enhance fuel-air

mixing and can add to the momentum of the relief gas discharge. The steam and air acts to

dilute the hydrocarbon fuel content, which also reduces the smoking tendency. The steam vapor

can also participate in the combustion kinetics, assisting in the conversion of carbon to carbon

monoxide.

The effective addition of steam from an upper steam ring increases the turbulence of

combustion. The overall noise level of the flare burner increases due to both the additional

combustion noise and jet noise from the steam nozzles. Steam-assisted, smokeless flares can

have significantly increased overall noise levels in comparison to flares with no steam assist.

The allowable flare burner exit velocity is a function of relief gas composition, flare burner

design, and the gas pressure available. These parameters are interrelated. Some flare tips

incorporate a flame-retention device or other means that provides a stable burning flame either

attached or detached relative to the flare tip. There is evidence that flame stability can be

maintained at relatively high velocities depending on the discharge properties and the type of tip

used.

Experience has shown that a properly designed and applied flare burner can have an exit

velocity of more than Mach 0.5, if pressure drop, noise, and other factors permit. Many pipe

flares, assisted or unassisted, and air-assisted flares have been in service for many years with

maximum Mach numbers of Mach 0.8 and higher.

Molecular Seal

- Air seals (also called purge reduction or gas seals) prevent air from entering the stack,

and are often recommended to prevent flashbacks and explosions in a flare system.

- Purge-reduction seals are not flame arrestors; that is, they will not stop a flashback.

They are designed as energy-conservation devices to reduce purge-gas flows required

to mitigate air infiltration into the stack.

- Air present in the stack can create a potentially explosive mixture with incoming flare gas

during low-fare gas flow rate conditions.

- There are two common types of mechanical seals, usually located at/or below the flare

tip that are used to reduce the amount of continuous purge gas required to prevent air

infiltration into the flare stack : molecular seals (sometimes called diffusion, buoyancy or

labyrinth seals), and velocity (fluidic) types.

- In the event of loss of purge-gas flow (and assuming that there is no other waste-gas

flow) the oxygen level below the velocity type seal will almost immediately begin to

increase. In the case of the buoyancy seal there is a time delay between the moment the

purge gas flow stops and the time the oxygen level below the seal begins to increase.

- The molecular seal consists of a baffled concentric cylinder arrangement, which uses the

difference in molecular weight of the flare gas and the ambient air to prevent air from

entering the flare stack. This seal normally reduces the purge-gas velocity required

through the tip to 0,003 m/s (0,01 ft/s).