PSV-2354-2360

Blocked Outlet Scenario

1.Determine relief load

The normal capacity of reformed gas is about 456508 kg/h and the rated capacity is

achieved by multiplying the normal capacity by 1.1 which equals to 502159 kg/h.

Remember since no PSV can discharge such load it is clear that more than 1 PSV is needed

So, accumulation of 16% is considered.

| First relief device | Additional relief devices | Accumulation |
| --- | --- | --- |
| 29barg | 31.4 | 16% |
| 100% | 105% | 116% |

2. Calculate orifice area

Determine if it is in critical flow:

If so, then:

Results

| T | 633K | W | 502159 kg/h |
| --- | --- | --- | --- |
| Z | 1 | A | 464 cm2 |
| M | 13.14 | A | 72 in2 |
| C | 0.0265 | Accumulation | 16% |

Note that the relieving temperature is obtained from Licensor EXCEL

Remember since no PSV can discharge such load it is clear that more than 1 PSV is needed

So, accumulation of 16% is considered.

5.Use API-526 to determine the designation and the inlet and outlet sizing

Licensor has divided the load between 6 PSV with the designation of 6Q8

Select proper PSV type by checking backpressure

According to licensor data, superimposed and build-up backpressure are max 12 barg. Since

the backpressure is constant and and more than 30% a balanced type could be selected

| superimposed | Build-up | Total |
| --- | --- | --- |
| 1.5 barg | 10.5 barg | 12barg |
| 5% | 36% | 41% |

Material Selection

Discussion

Licensor has selected 6 PSV with 6Q8 designation to handle such load to flare system. The

Licensor could have selected 5 PSV with 6R10 designation with total fluid of 550000 kg/h.

Remember PSV with T designation could not be selected since there is no rating for 600 Pound

for this PSV.