PSV-1031-1038-1043

Fire Scenario

Unwetted Surface

PSV SIZING PROCEDURE FOR UNWETTED FIRE SCENARIO

1.Determine the scenario, using API-521

2.Calculate the relief load, using API-520 Part1

3.Calculate the orifice area, using API-520 Part1

4.Select proper PSV type by checking backpressure

5.Use API-526 to determine the designation and the inlet and outlet sizing

6.Use API-520 Part2 to detail its construction

1.Determine the scenario, using API-521

Since it is R-1001 and exposed to fire then a fire scenario is defined.

| Parameters | Value | Parameters | Value |
| --- | --- | --- | --- |
| Diameter | 4.45 m | M | 16.54 |
| Height | 3.1 m | Set Pressure | 55 barg |
| Fluid | Natural Gas | Relieving Pressure | 67.55 bara |
| Z | 1.01 | Accumulation | 0.21 |
| Cp/Cv | 1.18 | Material | CS |

2.Calculate the relief load, using API-520 Part1

Calculation

| Parameters | Value | Parameters | Value |
| --- | --- | --- | --- |
| Aw | 65 m2 | KD | 0.975 |
| C9 | 0.2772 | F’ | 131 |
| C10 | 0.0395 | Relief Load | 3874 kg/hr |
| C | 0.0254 |  |  |

Note that since F’ is less than 182 the corresponding equation has been used in which F’ is set

to 182. Remember that in F’ calculation T1 is set to max 400 C

| Relief load | API-521 | Licensor | TCC |
| --- | --- | --- | --- |
| Value | 3874 | 1200 | 3825 |

3.Calculate the orifice area, using API-520 Part1

Use the following formula to calculate orifice area :

By setting F’, Aw, p1 to 132, 65 and 67500 kpa, we obtain 0.16 inch^2

5.Use API-526 to determine the designation and the inlet and outlet sizing

Since it is more than 0.11 inch and less than 0.196, then E is selected. Also, by checking its

rating and temperature limitation, 1E2 is selected.

Select proper PSV type by checking backpressure

According to licensor data, superimposed and build-up backpressure are max 1.5 and 18.5

barg. Since the backpressure is constant and the maximum backpressure is 36% a balanced

type could be selected.

| superimposed | Build-up | Total |
| --- | --- | --- |
| 1.5 barg | 18.5 barg | 20 barg |
| 2.7% | 33.6% | 36% |

Material Selection

Since it is NG, A216 WCB could be used for its body

Discussion

The selected PSV in site is 11/2F2 but calculation shows that it should be 1E2, then what is the

cause?

Topsoe has calculated 1200 kg/hr for its relief loading but my calculation and that of TCC show

otherwise.

Remember that according to API526 1E2 should be selected but TOPSOE has selected 11/2F2