PSV General Guideline

Chapter 1

Definitions and Terms

Maximum allowable working pressure (MAWP)

The maximum gauge pressure permissible at the top of a completed vessel in its normal

operating position at the designated coincident temperature specified for that pressure. The

pressure is the least of the values for the internal or external pressure as determined by the

vessel design rules for each element of the vessel using actual nominal thickness, exclusive of

additional metal thickness allowed for corrosion and loadings other than pressure. The MAWP is

the basis for the pressure setting of the pressure-relief devices that protect the vessel. The

MAWP is normally greater than the design pressure but can be equal to the design pressure

when the design rules are used only to calculate the minimum thickness for each element and

calculations are not made to determine the value of the MAWP.

Design pressure

Pressure, together with the design temperature, used to determine the minimum permissible

thickness or physical characteristic of each vessel component as determined by the vessel

design rules. The design pressure is selected by the user to provide a suitable margin above the

most severe pressure expected during normal operation at a coincident temperature. It is the

pressure specified on the purchase order.

This pressure may be used in place of the MAWP in all cases where the MAWP has not been

established. The design pressure is equal to or less than the MAWP. In most of the cases,

however, the pressure relieving device is set at the design pressure and not at the M.A.W.P.

because the latter is only known late in the design, when the detailed mechanical design of the

vessel is completed.

Accumulation

The pressure increases over the MAWP of the vessel, expressed in pressure units or as a

percentage of MAWP or design pressure. Maximum allowable accumulations are established by

applicable codes for emergency operating and fire contingencies.

Blow-down:

for gas 5-7% and for 2-phase and liquid is 20%

Overpressure:

is the pressure increase over the set pressure of the primary relieving device during

discharge. Overpressure is the same as accumulation only when the relieving device is

set to open at the maximum allowable working pressure of the vessel

Backpressure

The pressure that exists at the outlet of a pressure-relief device as a result of the

pressure in the discharge system. Backpressure is the sum of the superimposed and

built-up backpressures.

Built-up backpressure

The increase in pressure at the outlet of a pressure-relief device that develops as a

result of flow after the pressure-relief device opens.

Superimposed backpressure

The static pressure that exists at the outlet of a pressure-relief device at the time the

device is required to operate. Superimposed backpressure is the result of pressure in

the discharge system coming from other sources and may be constant or variable.

Relieving conditions

Relieving pressure, shown as P1 in the various sizing equations, is the inlet pressure of the

PRD at relieving conditions. The relieving pressure is the total of set pressure plus

overpressure. The examples cited in this section for the determination of relieving pressure refer

to PRVs; however, they are also applicable to non-reclosing PRDs (see Figure 15 and Figure 19

for pressure level relationships for these types of devices). The effects of inlet pressure drop on

specification of relieving pressure for PRV sizing can be neglected if the inlet pressure drops

does not exceed 3 % of set pressure.

Notes

1.In general, restrictions, either a specially designed spool piece or a restriction orifice, should

not be used as a means of limiting the capacity of a pressurization path.

2. When the increased operating condition will not exceed 10 hours at any one time or 100

hours per year, it is permissible to increase the pressure rating at the temperature existing

during the increased operating condition, by a maximum of 33 %.

3. When the increased operating condition will not exceed 50 hours at any one time or 500

hours per year, it is permissible to increase the pressure rating at the temperature existing

during the increased operating condition, by a maximum of 20 %.

Overall PSV Sizing Procedure

1.Determine the scenario, using API-521

2.Calculate the relief load, using API-520 Part1

3.Calculate the orifice area, using API-520 Part1

4.Select proper PSV type by checking backpressure

5.Use API-526 to determine the designation and the inlet and outlet sizing

6.Use API-521 Part2 to detail its construction

Chapter 2

Individual Overpressure Causes and Their Relieving Rates

Fire Scenario

Fire exposure on equipment can result in overpressure due to vapor generation (boiling of liquid

contents or decomposition reaction) and/or fluid expansion. Fire exposure can also cause

overheating of the vessel walls resulting in a reduction in material strength. Either the vapor

thermal-expansion-relief load or the boiling-liquid vaporization-relief load, but not both, should

be used. It is a practice that has been used for many years. There are no known  experimental

studies where separate contributions of vapor thermal expansion versus boiling-liquid

vaporization have been determined. When sizing the PRD for fire exposure, the contribution of

vaporizing liquid compared with vapor expansion is generally governing unless, for example, the

wetted surface has external insulation in accordance with 4.4.13.2.7 and the unwetted surfaces

are not insulated.

Effect of Fire on the Wetted Surface of a Vessel

To determine vapor generation, it is necessary to recognize only that portion of the vessel that is

wetted by its internal liquid and is equal to or less than 7.6 m (25 ft) above the source of flame.

Wetted surfaces higher than 7.6 m (25 ft) are normally excluded because pool fire flames are

not likely to impinge for long durations above this height.

Effect of Fire on the Unwetted Surface of a Vessel

Unwetted wall vessels are those that have no liquid in contact with the internal vessel walls (e.g.

internal walls are exposed only to a gas, vapor, or supercritical fluid or they are internally

insulated regardless of the contained fluids). These include vessels that contain separate liquid

and vapor phases under normal conditions but become single phase (above the critical) at

relieving conditions. Vessels can be designed to have internal insulation (e.g., refractory) and

such areas may be considered unwetted. A characteristic of a vessel with an unwetted internal

wall is that heat flow from the wall to the contained fluid is low as a result of the heat transfer

resistance of the contained fluid or any internal insulating material. Heat input from a fire to the

bare outside surface of an unwetted or internally insulated vessel can, in time, be sufficient to

heat the vessel wall to a temperature high enough to rupture the vessel.

Heat Absorption Equations for Vessels Containing Liquids

The amount of heat absorbed by a vessel exposed to an open fire is markedly affected by the

type of fuel feeding the fire, the degree to which the vessel is enveloped by the flames (a

function of vessel size and shape), the environment factor, firefighting, and drainage. Equation

(7) is used to evaluate these conditions if there are Prompt firefighting efforts and drainage of

flammable materials away from the vessels:

Where adequate drainage and firefighting equipment do not exist, Equation (8) should be used

Credit for thermal insulation is typically not taken because it usually does not meet the fire-

protection insulation requirements. Thermal insulation not equal to fire proof insulation.

The designer should be certain that any system of insulating materials permits the basic

insulating material to function effectively at temperatures up to 900 °C during a fire for up to 2

hr.

The value of thermal conductivity used in calculating the environmental factor credit for

insulation should be the thermal conductivity of the insulation at the mean temperature between

904°C and the process temperature expected at relieving conditions. (Use of a conservative

mean temperature of 1000°F (540°C) is suggested.)

For insulated vessels, the environment factor is given by:

Horizontal Drum

Up to 25 ft (7.6 m) above grade - Use total wetted vessel surface up to high liquid level.

Greater than 25 ft (7.6 m) above grade - Use the wetted area of the vessel surface to high liquid

level or up to the vessel center line whichever is less.

Vertical Drums

The wetted vessel surface within 25 ft (7.6 m) of grade, based on high liquid level, is used. If the

entire vessel is more than 25 ft (7.6 m) above grade, then only the surface of the bottom head

need be included. For vessels supported on skirts that do not require fireproofing of their inside

surface the surface of the bottom head need not be included in the wetted area regardless of

elevation.

Vertical Vessels

A wetted = 1.089 D 2 + π D h

Use this equation when the liquid surface elevation SE < 7.6 m. If the surface elevation of liquid

level is above 7.6 m, replace h by h - (SE – 7.6)

Fire Case Calculation by Software

Relief load calculation for single component:

Calculate Aw then calculate Q-fire then calculate landa then divide Q-fire to landa.

In order to calculate landa put Vf=0 and relieving pressure = 1.21pset + patm.

Relief load calculation for multi-component:

For multi-component systems, the latent heat of the residual liquid will change as the lighter

components are vaporized and removed from the system. In general, such systems require a

time-dependent analysis to determine the required relief area and the corresponding relief rate.

The following approach is suggested.

Using the composition of the residual liquid inventory in the vessel, perform a bubble point flash

at the accumulated pressure. In doing this flash, the flow rate of the feed stream to the flash can

be set at initial mass on the equipment. m=ρ.V

m: initial liquid mass in the equipment

V: liquid volume in the equipment up to HLL

ρ: liquid mass density at normal operation.

For each flash, sufficient heat is applied to vaporize a nominal 10 wt% of liquid, the remaining

liquid passing to the next flash.

An estimate of the time for each flash should be made.

time=required heat for each flash/calculated heat of fire.

A flash temperature of 400°C is obtained. The total vaporization time riches 2 hours. For

each stage, a latent heat of vaporization should be calculated and data for the flashed vapor

used to compute the value of a factor A’ which is indicative of the pressure relief valve orifice

area required based on that stage of the vaporization of the original fluid.

A =1/Y sqr(ZT/M)

pressure relief valve orifice area parameter (not the orifice area)

λ latent heat of vaporization (kJ/kg)

z vapor compressibility

T vapor temperature (K)

M vapor molecular weigh

Two-phase relief-device sizing is not normally required for the fire case, except for unusually

foamy materials or reactive chemicals.

If no accurate latent heat value is available for these hydrocarbons near the critical point, a

minimum value of 115 kJ/kg (50 Btu/lb) is sometimes acceptable as an approximation

Horizontal Vessels:

Liquid level below centerline

S=D Cos – 1 ((r- h) / r)

Liquid level above centerline

S=D {π-Cos – 1 ((h- r) / r)}

A wetted= (2.178D2+ πDL) (S/ πD)

As with vertical vessels, these equations are directly useful when the liquid surface elevation S

E < 7.6 m. If the surface elevation of liquid level is above 7.6 m, replace h by h - (S E – 7.6).

Trayed column

High liquid level in bottom plus liquid holdup from all trays. Level in reboiler is to be included if

the reboiler is an integral part of the column. Total wetted surface up to the height of 7.6m.

Vessel heads protected by support skirts with limited ventilation are not normally included as

wetted surface area. Liquid hold-up on each tray shall be equal to the weir height plus 50mm.

Air-Coolers

It is not necessary to consider the bare area for air-cooled condensers, whether partial or total

condensing, as long as both of the following conditions are satisfied:

1. The tubes are sloped so that they are self-draining.

2. There is no control valve or pump connected directly to the condenser

liquid outlet.

Heat Absorption Equations for Vessels Containing Only Gases, Vapors, or Supercritical

Fluids

The discharge areas for PRDs on vessels containing supercritical fluids, gases or vapors

exposed to open fires can be estimated using Equation (9). In certain cases, the normal

operating pressure can be below the thermodynamic critical conditions but the relieving

pressure is supercritical. In such cases, the guidance below can be used to size the relief

device.

In the use of Equation (9), no credit has been taken for insulation:

The derivations of Equations (9), (10), (13), and (14) [53] are based on the physical properties

of air and the perfect gas laws. The derivations assume that the vessel is uninsulated and has

no mass, that the vessel wall temperature does not reach rupture stress temperature, and that

there is no change in fluid temperature. These assumptions should be reviewed to ensure that

they are appropriate for any particular situation. Insulation that meets the external insulation

criteria outlined in 4.4.13.2.7 offers a mitigating benefit when gas-filled vessels are exposed to a

fire by decreasing the rate at which the metal wall temperature rises.

The surface area potentially exposed to a fire should be used when determining the fire-relief

requirements of gas-filled vessels.

Failure of Automatic Controls Scenario

Inadvertent Valve Opening

The inadvertent opening of any valve from a source of higher pressure, such as high-pressure

steam or process fluids, should be considered. Administrative controls can be used to prevent

inadvertent valve opening, subject to 4.2.1 and 4.2.2. The relief load should be determined

using the maximum operating pressure upstream of the valve and the relieving pressure on

equipment downstream of the valve. If the pressure source is a pipeline or a production well, the

pressure upstream of the valve may reach the maximum shut-in pressure of the source after a

shutdown. The user should determine whether inadvertent valve opening combined with

maximum shut-in pressure in upstream system is a credible relief case. There can be single or

multiple inlet lines fitted with control devices. The scenario to consider is that one inlet valve is in

a fully opened position regardless of the control valve failure position. Opening of this control

valve can be caused by instrument failure or misoperation. If the system has multiple inlets, the

position of any control device in those remaining lines shall be assumed to remain in its normal

operating position. Therefore, the required relieving rate is the difference between the maximum

expected inlet flow and the normal outlet flow, adjusted for relieving conditions and considering

unit turndown, assuming that the other valves in the system are still in operating position at

normal flow (i.e., normally open, normally closed, or throttling.

The following applies when a manual or actuated valve is inadvertently opened, causing

pressure buildup in a vessel. The vessel should have a PRD large enough to pass a rate equal

to the flow through the open valve; credit may be taken for the flow capacity of vessel outlets

that can reasonably be expected to remain open. The manual or actuated valve should be

considered as passing its capacity at a full-open position with the pressure in the vessel at

relieving conditions. Volumetric or heat-content equivalents may be used if the manual or

actuated valve admits a liquid that flashes or a fluid that causes vaporizing of the vessel

contents.

An important consideration is the effect of having a manual bypass on the inlet control valve(s)

at least partially open. If, during operation, the bypass valve is opened to provide additional flow,

then this total flow (control valve wide open and bypass valve normal position) shall be

considered in the relieving scenario. The potential for the bypass valve to be inadvertently

opened (e.g. during normal operations, control valve maintenance, start-up, shutdown, or

special operations) while the control valve is operating (both bypass and control valve wide

open) should also be considered. Administrative controls can be used to prevent inadvertent

opening of the bypass valve. The user is cautioned that some systems can have unacceptable

risk due to failure of administrative controls and resulting consequences due to loss of

containment. In these cases, limiting the overpressure to the normally allowable overpressure

can be more appropriate. Note that the entire system, including all of the auxiliary devices (e.g.

gasketed joints, instrumentation), should be considered for the overpressure during the failure of

administrative controls.

Other situations can arise where problems involved in evaluating relief requirements after the

failure of an inlet control device are more complex and of special concern (e.g. a pressure

vessel operating at a high pressure where liquid bottoms are on level control and discharge into

a lower-pressure system).

Usually, when the liquid is let down from the high-pressure vessel into

the low-pressure system, only the flashing effect is of concern in the event that the low-pressure

system has a closed outlet. However, the designer should also consider that vapors flow into

the low-pressure system if loss of liquid level occurs in the vessel at higher pressure. In this

case, if the volume of the source of incoming vapors is large compared with the volume of the

low-pressure system or if the source of vapor is unlimited, serious overpressure can rapidly

develop. When this occurs, it can be necessary to size relief devices on the low-pressure

system to handle the full vapor flow through the liquid control valve. The loss of liquid level

followed by high-pressure vapor flow is commonly referred to as “vapor breakthrough” or “gas

blowby.

Closed Outlets

The inadvertent closure of a valve on the outlet of pressure equipment while the equipment is

on stream can expose the equipment to a pressure that exceeds the MAWP. Every valve (i.e.

manual, control, or remotely operated) should be considered as being subject to inadvertent

operation. If closure of an outlet valve can result in pressure in excess of that allowed by the

design code, a PRD is required. If the equipment is designed to the maximum source pressure,

then closure of an outlet valve will not result in overpressure, so a PRD is not required for the

closed outlet scenario. Additional Considerations Involving Pumps: The system does not require

relief protection for the closed outlet (i.e. shut-in) scenario if the pump, piping, and other

equipment downstream of a centrifugal pump (which can be exposed to the shut-in pressure of

the pump, e.g. closed block valve in the discharge system) are designed to withstand the

maximum shut-in pressure of the pump or other pressure source such as a start-up or warm-up

line. For positive displacement pumps, pressure-relief protection is usually required to protect

the pump itself and downstream equipment against shut-in conditions.

In the case of a manual valve, administrative controls can be used to prevent the closed

outlet scenario unless the resulting pressure exceeds the maximum allowed by the

pressure design code [usually the corrected hydrotest pressure is exceeded.

Liquid Overfill

Liquid overfills of a vessel results in a closed outlet case. As an alternative to providing an

adequacy sized pressure relief valve, the following alternative options are available for

preventing overpressure:

1) Increase the system design pressure and/or pressure relief valve set pressure within code

allowances.

2) Provide a safety instrument system (HIPS) to isolate the inflow on increasing vessel

inventory.

3) Rely on operator intervention to prevent overfill from occurring. Where liquid overfill is

eliminated on the basis of operator intervention, at least one additional and independent alarm

shall be provided to indicate that the situation has not yet been brought under control. This

alarm may monitor a parameter other than level (e.g.differential pressure)

Sizing Procedure for Gases

Determine if it is in critical flow:

If so, then:

If it is subcritical flow, then use the following equation for conventional or pilot type

k

Hydraulic Expansion

Hydraulic expansion is the increase in liquid volume caused by an increase in temperature (see

Table 2). It can result from several causes, the most common of which are the following.

a) Piping or vessels are blocked in while they are filled with cold liquid and are subsequently

heated by heat tracing, coils, ambient heat gain, or fire.

b) A heat exchanger is blocked in on the cold side with flow in the hot side.

Caution—Block valves have the potential to leak, thereby admitting either cold or hot fluid into a

heat exchanger that is intended to be blocked in, resulting in a potential overpressure.

c) Piping or vessels are blocked in while they are filled with liquid at near-ambient temperatures

and are heated by direct solar radiation.

Where the system under consideration for thermal relief consists of piping only (does not contain

Pressure vessels or heat exchangers), a PRD might not be required to protect piping from thermal

expansion if any of the following:

a) the piping always contains a pocket of noncondensing vapor, such that it can never become

liquid-full; or Caution—Small vapor or gas pockets can disappear upon heating due to

compression and/or solubilization. In contrast, multicomponent mixtures with a wide boiling range

can always have sufficient vapor present to preclude becoming completely liquid-full. The liquid-

volume change upon solar heating, heat tracing, heating to ambient temperature, or heat from

another source should be estimated to determine if the volume of the vapor pocket is sufficient

for liquid expansion.

b) the piping is in continuous use (i.e. not batch or semicontinuous use) and drained after being

blocked in using well supervised procedures or permits; or

c) the fluid temperature is greater than the maximum temperature expected from solar heating

[usually approximately 60 °C to 70 °C (approximately 140 °F to 160 °F)] and there are no other

heat sources such as heat tracing (note that fire is generally not considered when evaluating

pressure-relief requirements for piping);

d) the estimated pressure rise from thermal expansion is within the design limits of the equipment

or piping.

Sizing and Set Pressure

Since every application is for a relieving liquid, the required relieving rate is small; specifying an

oversized device is, therefore, reasonable. A nominal diameter (DN) 20 × DN 25 (NPS 3/4 × NPS

1) relief valve is commonly used.

If there is reason to believe that this size is not adequate, the procedure in 4.4.12.3 can be applied.

The thermal-relief pressure setting should never be above the maximum pressure permitted by

the weakest component in the system being protected. However, the PRD should be set high

enough to open only under hydraulic expansion conditions. If thermal-relief valves discharge into

a closed system, the effects of backpressure should be considered.

Two general applications for which thermal-relieving devices larger than a DN 20 × DN 25 (NPS

3/4 × NPS 1) valve can be required are long pipelines of large diameter in uninsulated,

aboveground installations and large vessels or heat exchangers operating liquid-full. Long

pipelines can be blocked in at or below ambient temperature; the effect of solar radiation raises

the temperature at a calculable rate. If the total heat transfer rate and thermal expansion

coefficient for the fluid are known, a required relieving rate can be calculated. See Parry [135] for

additional information on thermal relief

In order to calculate the orifice area use the following equation:

Notes:

When a PRV is sized for viscous liquid service, it should first be sized as if it were for a

non-viscous type application (i.e. Kv = 1.0) so that a preliminary required discharge area, AR,

can be obtained from Equation (32) or Equation (33). The user is cautioned that the equations

presented here may not be applicable for non-Newtonian applications (see 5.10). From the list

of API 526 standard orifice sizes or the manufacturer’s listing, the next orifice size, A selected,

larger than AR should be chosen to calculate the Reynolds number, Re, from either of the

following relationships

After the Reynolds number, Re, is determined, the factor Kv is obtained and applied in Equation

(32)or Equation (33) to correct the preliminary required discharge area, AR. If the corrected area

exceeds the chosen standard orifice area, A selected, the Reynolds number should be

recalculated using the next larger standard orifice size and the value of Kv updated accordingly.

Selecting PSV Type based on Backpressure

Regardless of whether the valve is vented directly to atmosphere or the discharge is piped to a

collection system, the backpressure may affect the operation of the PRV. Effects due to

backpressure may include variations in opening pressure, reduction in flow capacity, instability,

or a combination of all three. backpressure can be constant if the valve outlet is connected to a

process vessel or system that is held at a constant pressure. In most cases, however, the

superimposed backpressure will be variable as a result of changing conditions existing in the

discharge system.

Effects of Superimposed Backpressure on Pressure-relief Valve Opening

A pilot-operated PRV consists of the main valve, which normally encloses a floating unbalanced

piston assembly, and an external pilot (see Figure 10 through Figure 14). The piston is designed

to have a larger area on the top than on the bottom. Up to the set pressure, the top and bottom

areas are exposed to the same inlet operating pressure. Because of the larger area on the top

of the piston, the net force holds the piston tightly against the main valve nozzle. As the

operating pressure increases, the net seating force increases and tends to make the valve

tighter. This feature allows most pilot-operated valves to be used where the maximum expected

operating pressure is higher than the percentage shown in Figure 15. At the set pressure, the

pilot vents the pressure from the top of the piston; the resulting net force is now upward causing

the piston to lift, and process flow is established through the main valve. After the overpressure

incident, the pilot will close the vent from the top of the piston, thereby re-establishing pressure,

and the net force will cause the piston to reseat. The pilots may be either a flowing or

nonflowing type. The flowing type allows process fluid to continuously flow through the pilot

when the main valve is open; the nonflowing type does not. The nonflowing pilot type is

generally recommended for most services to reduce the possibility of hydrate formation (icing)

or solids in the lading fluid affecting the pilot’s performance.

The modulating pilot, as shown in Figure 17, opens the main valve only enough to satisfy the

required relieving capacity and can be used in gas, liquid, or two-phase flow applications. A

modulating pilot-operated valve, in contrast to a pop-action valve, limits the amount of relieving

fluid to only the amount required to prevent the pressure from exceeding the allowable

accumulation. Since a modulating pilot only releases the required relieving rate, the calculation

of built-up backpressure may be based on the required relieving rate instead of the rated

capacity of the valve corrected for the actual overpressure. The modulating pilot valve also can

reduce interaction with other pressure control equipment in the system during an upset

condition, reduce unwanted atmospheric emissions, and reduce the noise level associated with

discharge to the atmosphere.

PRV Instability

PRV Cycling

PRV Flutter

PRV Chatter

Chattering is where the PRV opens and closes at a very high frequency (on the order of the

natural frequency of the valve’s spring/mass system). Spring-loaded PRVs are spring/mass

devices and consequently are susceptible to dynamic interaction with the system. The primary

concern is loss of containment (loosening of flange bolts or failure of piping components due to

fatigue) caused by pressure pulsation or impact loading from rapid hammering of the valve disk

onto the valve seat. Chattering may lead to significantly reduced PRV flow capacity. As a

secondary effect, the chattering can cause valve seat damage and mechanical failure of valve

internals (galling and bellows failure). Spring-loaded PRVs and pop action pilot valves can

experience chatter. Modulating pilot-operated or remote sensing pop-action pilot PRVs are less

likely to chatter). The damaging forces on piping associated with fluid pressure and velocity

changes associated with chatter are much more severe in liquid service as compared to vapor

service due to the higher densities associated with liquids. This is supported by analysis that

shows that the pressure change as a result of fluid acceleration is typically small in inlet piping

applications in vapor service.

cycling

The relatively low frequency (a few cycles per second to a few seconds per cycle) opening and closing of

a pressure-relief valve.

flutter

The abnormal, rapid reciprocating motion of the moveable parts of a pressure-relief valve, during which

the disk does not contact the seat or the upper stop.

Potential Causes of PRV Instability

1.Excessive PRV Inlet Pressure Loss

2. Excessive Built-up Backpressure

3. Acoustic Interaction

4. Retrograde Condensation

5. Improper Valve Selection

6. Oversized PRVs

PRD Location

If other factors permit, the PRD should normally be placed close to the protected equipment or

system of equipment so that the pressure in the protected equipment stays within code

allowable limits and to avoid PRV instability. The PRD should not be located where there are

pressure fluctuations large enough to result in relief valve simmering/activation or rupture disk

fatigue. On installations that have pressure fluctuations that peak close to the set pressure of

the PRV or burst pressure of a rupture disk, the PRD should be located farther from the source

and in a more stable pressure region.

— locations close to control valves, other valves, and other appurtenances;

— locations close to orifice plates and flow nozzles;

— locations close to other fittings, such as short radius elbows;

— locations close to the discharge of positive displacement pumps or compressors.

The potential effect of pressure fluctuations on the relief device may be reduced by the following

— locating the PRD 10 or more pipe diameters from any areas as described above;

— providing a well-rounded, smooth branch connection where the relief device inlet piping joins

the main piping run.

— providing a larger branch connection (relative to the size of the PRV inlet).

The PRD inlet and outlet piping should be free-draining (no pockets) away from the PRD.

The nominal size of the inlet piping and fittings shall be the same as or larger than the nominal

size of the pressure-relief valve inlet connection.

Horizontal lines are generally regarded as self-draining. However, avoid the installation of a

PRV at the end of a long horizontal inlet pipe through which there is normally no flow. Solids,

such as rust or scale, may accumulate, or liquid may be trapped, creating interference with the

valve’s operation or requiring more frequent valve maintenance.

Auto-refrigeration during discharge can cool the outlet of the PRD and the discharge piping to

the point that brittle fracture can occur. Piping design, including material selection, shall

consider the expected discharge temperature.

Note that the rated capacity corrected for the actual overpressure can vary depending on the

overpressure scenario common relief header piping in closed discharge systems can be sized

using the protected system’s required relieving capacity.

For a modulating pilot-operated PRV, the discharge piping can be sized using the required

relieving capacity of the system that the valve is protecting.

Whenever the atmospheric vent, discharge piping, or common relief header piping is sized

using the system’s required relieving capacity instead of the rated capacity of the valve

corrected for the actual overpressure, the backpressure should be re-checked whenever

changes are made to the process that affect the required relieving capacity of the system the

valve is protecting. The discharge piping from thermal relief valves designed solely to protect

against liquid hydraulic expansion due to ambient heating (including solar radiation) typically

does not need to be sized to meet the built-up backpressure limits provided in API 520, Part I

and as discussed in 6.3.1. The reason for this is that the capacity of these PRVs is typically

larger by an order of magnitude (>10 times) than the required relief rate and the flow in the

discharge line never reaches a steady state flow at the capacity. Superimposed backpressure

that exceeds the inlet pressure of a pilot-operated PRV can cause the main valve to open,

allowing reverse flow through the main valve.

PRV Inlet Pressure Drop Limitations

1.Confirm the inlet pressure losses do not significantly affect the capacity of the PRV.

2.Confirm the PRV is set to open at or below the maximum allowable working pressure for all

equipment being protected.

3.Limit the pressure to the maximum allowable accumulation for all equipment being protected

4.Provide reasonable assurance that the inlet pressure losses are unlikely to result in

destructive instability of the PRV.

PRV Inlet Pressure Loss Criteria

The total nonrecoverable pressure loss between the protected equipment and the pressure-

relief valve should not exceed 3 % of the PRV set pressure, except as noted below:

— thermal relief valves

— remotely sensed pilot-operated relief valves

— an engineering analysis is performed for the specific installation.

Note that keeping the pressure loss below 3 % becomes progressively more difficult at low

pressures and/or as the orifice size of a PRV increases. In certain applications, it is difficult to

meet the 3 % criterion for the largest API 526 [2] orifice size for a given inlet flange diameter

(e.g., 2J3, 4P6, 6R8, etc.). There are some non-API 526 valves that also exhibit this behavior.

When a pressure-relief valve is installed on a normally flowing process line, the 3 % limit should

be applied to the sum of the loss in the normally nonflowing PRV inlet pipe and the incremental

pressure loss in the process line caused by the flow through the PRV.

Isolation Valve Requirements

Isolation (stop) valves are allowed upstream and/or downstream of the pressure-relieving device

for the purpose of inspection, testing, and repair of the pressure-relieving device or discharge

header isolation. For outlet isolation valves, to help minimize the built-up backpressure, the flow

area in the outlet isolation valve should be equal to or greater than the outlet area of the PRV.

Typically, the inlet isolation valves for spare relief devices are closed and the outlet isolation

valves are open. The outlet isolation valve for spare relief devices can be closed during

operation if exposure to the fluid is a concern; however, the pressure temperature rating of the

PRD outlet, the outlet isolation valve, and intervening piping should be suitable for the

conditions upstream of the relief device in case of leakage. Protection of the PRD from

discharge system fluids without closing the outlet isolation valve can also be achieved either by

providing a purge or by installing a discharge rupture disk.

Rupture Disk

Rupture disk devices are non-reclosing PRDs used to protect vessels, piping, and

Other pressure-containing components from excessive pressure and/or vacuum. Rupture disks

are used in single and multiple relief device installations. They are also used as redundant

PRDs. With no moving parts, rupture disks are simple, reliable, and faster acting than other

PRDs. Rupture disks react quickly enough to relieve some types of pressure spikes. Because of

their light weight, rupture disks can be made from high alloy and corrosion resistant materials

that are not practical in PRVs.

Rupture disks can be specified for systems with vapor (gas) or liquid pressure-relief

requirements. Also, rupture disk designs are available for highly viscous fluids. The use of

rupture disk devices in liquid service should be carefully evaluated to ensure that the design of

the disk is suitable for liquid service. The user should consult the manufacturer for information

regarding liquid service applications.

Rupture disk devices often have different opening characteristics as a function of the fluid state

against the disk at the time of bursting. To account for the resulting differences in the resistance

to flow, certified Kr values are stated in terms of Krg (gas), Krl (liquid), or Krgl (gas or liquid). In

application, use the following guidelines.

1.When the fluid initiating rupture (in contact with rupture disk) is compressible, rupture disks

rated with Krg or Krgl should be used.

2.When the fluid initiating rupture (in contact with rupture disk) is incompressible, rupture disks

rated with Krl or Krgl should be used.

The rupture disk is also a temperature-sensitive device. Burst pressures can vary significantly

with the temperature of the rupture disk device. This temperature may be different from the

normal fluid operating temperature. As the temperature at the disk increases, the burst pressure

usually decreases. Since the effect of temperature depends on the rupture disk design and

material, the manufacturer should be consulted for specific applications. For these reasons, the

rupture disk shall be specified at the pressure and temperature the disk is expected to burst.

Application of Rupture Disks

Rupture disks can be used in any application requiring overpressure protection where a non-

reclosing device is suitable. This includes single, multiple, and fire applications as specified in

UG-134 of the ASME Code.

Rupture Disk Device at the Inlet of a Pressure-relief Valve

Rupture disks are used upstream of PRVs to seal the system to meet emissions standards, to

provide corrosion protection for the valve, and to reduce valve maintenance.

When a rupture disk device is installed at the inlet of a PRV, the devices are considered to be

close coupled, and the specified burst pressure and set pressure should be the same nominal

value.

When installed in liquid service, it is especially important for the disk and valve to be close

coupled to reduce shock loading on the valve The space between the rupture disk and the PRV

shall have a free vent, pressure gauge, trycock, or suitable telltale indicator as required in UG-

127 of the ASME Code. Users are warned that a rupture disk will not burst in tolerance if

backpressure builds up in a nonvented space between the disk and the PRV, which will occur

should leakage develop in the rupture disk due to corrosion or other cause (

Rupture Disk Device at the Outlet of a Pressure-relief Valve

A rupture disk device may be installed on the outlet of a PRV to protect the valve from

atmospheric or downstream fluids. Consideration should be given to the valve design so

that it will open at its proper pressure setting regardless of any backpressure that may

accumulate between the valve and rupture disk.

Types of Rupture Disks

There are three major rupture disk types

1. Forward-acting, tension-loaded

2. Reverse-acting, compression-loaded;

3. Graphite, shear-loaded.

Forward-acting Solid Metal Rupture Disks

A forward-acting rupture disk is a formed (domed), solid metal disk designed to burst at a rated

pressure applied to the concave side (see Figure 21). This rupture disk typically has an angular

seat design and provides a satisfactory service life when operating pressures are up to 70 % of

the marked burst pressure of the disk (70 % operating ratio). Consult the manufacturer for the

actual recommended operating ratio for the specific disk under consideration. If vacuum or

backpressure conditions are present, the disk can be furnished with a support to prevent

reverse flexing. These disks have a random opening pattern and are considered fragmenting

designs that are not suitable for installation upstream of a PRV.

Forward-acting Scored Rupture Disks

The scored forward-acting rupture disk is a formed (domed) disk designed to burst along scored

lines at a rated pressure applied to the concave side (see Figure 22). Some designs provide

satisfactory service life when operating pressures are up to 85 % to 90 % of the marked burst

pressure of the disk (85 % to 90 % operating ratio). Most designs withstand vacuum conditions

without a vacuum support. If backpressure conditions are present, the disk can be furnished

with a support to prevent reverse flexing. Because the score lines control the opening pattern,

this type of disk can be manufactured to be nonfragmenting and is acceptable for installation

upstream of a PRV. The scored,forward-acting rupture disk is manufactured from thicker

material than nonscored designs with the same burst pressure, and it provides additional

resistance to mechanical damage.

Forward-acting Scored Rupture Disks

A forward-acting composite rupture disk is a flat or domed multipiece construction disk (see

Figure 23). The domed composite rupture disk is designed to burst at a rated pressure applied

to the concave side. The flat composite rupture disk may be designed to burst at a rated

pressure in either or both directions. Some designs are nonfragmenting and acceptable for use

upstream of a PRV. A flat composite rupture disk is available for the protection of low-pressure

vessels or the isolation of equipment such as exhaust headers or the outlet side of a PRV. This

disk usually comes complete with gaskets and is designed to be installed between companion

flanges rather than within a specific rupture disk holder. Flat composite rupture disks generally

provide satisfactory service life when operating pressures are 50 % or less of the marked burst

pressure (50 % operating ratio). The slits and tabs in the top section provide a predetermined

opening pattern for the rupture disk. If vacuum or backpressure conditions are present,

composite disks can be furnished with a support to prevent reverse flexing (see Figure 23). A

domed, composite rupture disk generally provides satisfactory service life when the operating

pressure is 80 % or less of the marked burst pressure (80 % operating ratio).

A flat composite rupture disk is available for the protection of low-pressure vessels or the

isolation of equipment such as exhaust headers or the outlet side of a PRV. This disk usually

comes complete with gaskets and is designed to be installed between companion flanges rather

than within a specific rupture disk holder. Flat composite rupture disks generally provide

satisfactory service life when operating pressures are 50 % or less of the marked burst pressure

(50 % operating ratio).

Reverse-acting Rupture Disks

A reverse-acting rupture disk typically is a formed (domed) solid metal disk designed to

reverseand burst at a rated pressure applied on the convex side. Reverse-acting rupture disks

are designed to open by such methods as shear knife blades, tooth rings, or scored lines

Reverse-acting rupture disks may be manufactured as nonfragmenting and are suitable for

installation upstream of PRVs. These disks provide satisfactory service life when operating

pressures are 90 % or less of marked burst pressure (90 % operating ratio). Some types of

reverse-buckling disks are designed to be exposed to pressures up to 95 % of the marked burst

pressure. Consult the manufacturer for the actual recommended operating ratio for the specific .

disk under consideration. Because a reverse acting rupture disk is operated with pressure

applied on the convex side, thicker disk materials may be used, thereby lessening the effects of

corrosion, eliminating the need for vacuum support, and providing longer service life under

pressure/vacuum cycling conditions and pressure fluctuations

Graphite Rupture Disks

Graphite rupture disks are typically machined from a bar of fine graphite that has been

impregnated with a sealing compound to seal the porosity of the graphite matrix (see Figure 26).

The disk operates on a pressure differential across the center diaphragm or web portion of the

disk. Graphite rupture disks provide a satisfactory service life when operating pressures are up

to 80 % of the marked burst pressure (80 % operating ratio) and can be used in both liquid and

vapor service.

If vacuum or backpressure conditions are present, the disk can be furnished with a support to

prevent reverse flexing. These disks have a random opening pattern and are considered

fragmenting designs that are not suitable for installation upstream of a PRV. A metallic ring

called armoring is often added to the outside diameter of the disk to help support uneven piping

loads and minimize the potential for cracking of the outer graphite ring and blowout of process

fluid.

Rupture Disk Selection

the user is cautioned to make sure that the upper limit of the manufacturing design range does

not exceed the MAWP of the equipment being protected.

Selection Procedure

Sizing Procedure