PSV Sizing Notes

- Standards:

API STD 520 for sizing, selection and installation of pressure relieving devices

API STD 521 for pressure relieving and depressurizing system

- Procedure

- Design pressure is the most severe condition of coincident internal or external pressure and temperature (minimum or maximum) that results in the greatest required component thickness and the highest component rating

- In most of the cases, however, the pressure relieving device is set at the design pressure and not at the M.A.W.P. because the latter is only known late in the design, when the detailed mechanical design of the vessel is completed.

- Pressure Relief Valve: is an automatic pressure-relieving device actuated by the static pressure upstream of the valve, and which opens in proportion to the increase in pressure over the opening pressure. It is used primarily for liquid service.

- Pressure Safety Valve: is an automatic pressure-relieving device actuated by the static pressure upstream of the valve and characterized by rapid full opening or pop action. It is used for gas or vapor service. (In the petroleum industry it is used normally for steam or air.)

- Contingency: An abnormal event which is the cause of an emergency condition

- MAWP >= DP

- Set Pressure: is the inlet pressure at which the pressure relief valve is adjusted to open under service conditions.

- Accumulation: The pressure increases over the maximum allowable working pressure of the vessel, expressed in pressure units or as a percentage of maximum allowable working pressure (MAWP) or design pressure. Maximum allowable accumulations are established by applicable codes for emergency operating and fire contingencies.

- Overpressure: is the pressure increase over the set pressure of the primary relieving device during discharge. Overpressure is the same as accumulation only when the relieving device is set to open at the maximum allowable working pressure of the vessel.

- Relieving Pressure: is equal to the valve set pressure (or rupture disk burst pressure) plus the overpressure.

is the difference between the set pressure and the reseating pressure of a pressure relief valve, expressed as percent of set pressure?

- Superimposed Back Pressure: is the pressure at the outlet of the pressure relief valve while the valve is in a closed position. This type of back pressure comes from other sources in the discharge system; it may be constant or variable; and it may govern whether a conventional or balanced bellows valve should be used in specific applications

- Built-up Backpressure: The increase in pressure at the outlet of a pressure relief device that develops as a result of flow after the pressure relief device opens.

- In general, restrictions, either a specially designed spool piece or a restriction orifice, should not be used as a means of limiting the capacity of a pressurization path.

- Blow-down : for gas 5-7% and for 2-phase and liquid is 20%

- In case of evaluating the effects of operator response for the study to decide the maximum relieving load, it should be considered that the response time between 10 and 30 minutes for operator to take appropriate action ,depending on the complexity of the plants, is required.

- When the increased operating condition will not exceed 10 hours at any one time or 100 hours per year, it is permissible to increase the pressure rating at the temperature existing during the increased operating condition, by a maximum of 33 %

- When the increased operating condition will not exceed 50 hours at any one time or 500 hours per year, it is permissible to increase the pressure rating at the temperature existing during the increased operating condition, by a maximum of 20 %

- Causes of Overpressure:

Closed (Blocked) outlet  External fire  Inadvertent control valve opening  Check-valve malfunction  Utility failure  Power  Cooling water  Instrument air  Steam  Fuel gas (Fuel oil)  Thermal expansion  Tube rupture of heat exchanger  Abnormal process condition : For example, runaway reaction, and so on  Equipment failure : Fans, Compressor, Pump, Blower, and so on  Column Relief Scenarios (Reflux Failure, Power Failure, etc.)

- Closed outlet could be caused by:

Downstream control valve fails closed

Isolation valve inadvertently closed by operator

Chemical reactions create a flow blockage

Source pressure exceeds downstream equipment design pressure. Sources are:

Pumps

Compressors

High pressure utilities

High pressure upstream fluids

- External Fire

All pressure vessels shall be protected from overpressure due to vaporization or expansion of contained fluid caused by exposure to fire by use of a pressure relief valve

In some cases of fire may heat the walls of pressure vessel to temperature

far beyond the specified metal design temperature, creating a real potential

for the vessel to fail at a pressure below the set pressure of the pressure relief

valve.

To determine vapor generation, it is necessary to recognize only that

portion of the vessel that is wetted by its internal liquid and is equal

to or less than 7,6 m (25 ft) above the source of flame

Either the vapor thermal-expansion relief load or the boiling-liquid

relief load, but not both, should be used. It is a practice

that has been used for many years.

Credit for thermal insulation is typically not taken because it usually does not meet the fire-protection insulation requirements.  Thermal insulation not equal to fire proof insulation.

The designer should be certain that any system of insulating materials permits the basic insulating material to function effectively at temperatures up to 900 °C during a fire for up to 2 hr.

The value of thermal conductivity used in calculating the environmental factor

credit for insulation should be the thermal conductivity of the insulation

at the mean temperature between 904°C and the process temperature

expected at relieving conditions. (Use of a conservative mean temperature

of 1000°F (540°C) is suggested.)

For insulated vessels, the environment factor is given by :

+Horizontal drum

Up to 25 ft (7.6 m) above grade - Use total wetted vessel surface up to

high liquid level.

Greater than 25 ft (7.6 m) above grade - Use the wetted area of the

vessel surface to high liquid level or up to the vessel center line whichever is

less.

+ Vertical Drums

The wetted vessel surface within 25 ft (7.6 m) of grade, based on high liquid

level, is used. If the entire vessel is more than 25 ft (7.6 m) above grade,

then only the surface of the bottom head need be included. For vessels

supported on skirts that do not require fireproofing of their inside surface the

surface of the bottom head need not be included in the wetted area

regardless of elevation.

Vertical Vessels

A wetted = 1.089 D 2 + π D h

Use this equation when the liquid surface elevation SE < 7.6 m. If the

surface elevation of liquid level is above 7.6 m, replace h by h - (SE – 7.6)

Horizontal Vessels:

# Liquid level below centerline

S=D Cos – 1 ((r- h) / r)

#  Liquid level above centerline

S=D {π-Cos – 1 ((h- r) / r)}

A wetted=(2.178D2+ πDL)(S/ πD)

As with vertical vessels, these equations are directly useful when the liquid

surface elevation S E < 7.6 m. If the surface elevation of liquid level is above

7.6 m, replace h by h - ( S E – 7.6).

- Trayed column

High liquid level in bottom plus liquid holdup from all trays. Level in reboiler is to be included if the reboiler is an integral part of the column. Total wetted surface up to the height of 7.6m. Vessel heads protected by support skirts with limited ventilation are not normally included as wetted surface area. Liquid hold-up on each tray shall be equal to the weir height plus 50mm.

- Air-Coolers

It is not necessary to consider the bare area for air-cooled condensers, whether

partial or total condensing, as long as both of the following conditions are satisfied:

1. The tubes are sloped so that they are self-draining.

2. There is no control valve or pump connected directly to the condenser

liquid outlet.

- Relief load calculation for single component:

Calculate Aw then calculate Q-fire then calculate landa then divide Q-fire to landa.

In order to calculate landa put Vf=0 and relieving pressure = 1.21pset + patm.

- Relief load calculation for multi-component:

For multi-component systems, the latent heat of the residual liquid will change as the lighter components are vaporized and removed from the system. In general, such systems require a time-dependent analysis to determine the required relief area and the corresponding relief rate. The following approach is suggested.

Using the composition of the residual liquid inventory in the vessel, perform a bubble point flash at the accumulated pressure. In doing this flash, the flow rate of the feed stream to the flash can be set at initial mass on the equipment. m=ρ.V

m: initial liquid mass in the equipment

V: liquid volume in the equipment up to HLL

ρ: liquid mass density at normal operation.

For each flash, sufficient heat is applied to vaporize a nominal 10 wt% of liquid, the remaining liquid passing to the next flash.

An estimate of the time for each flash should be made.

time=required heat for each flash/calculated heat of fire.

A flash temperature of 400°C is obtained. The total vaporization time riches 2 hours

For each stage, a latent heat of vaporization should be calculated and data for the flashed vapor used to compute the value of a factor A’ which is indicative of the pressure relief valve orifice area required based on that stage of the vaporization of the original fluid.

A =1/Y sqr(ZT/M)

pressure relief valve orifice area parameter (not the orifice area)

λ latent heat of vaporization (kJ/kg)

z vapor compressibility

T vapor temperature (K)

M vapor molecular weigh

- Two-phase relief-device sizing is not normally required for the fire case, except for unusually foamy materials or reactive chemicals.

- If no accurate latent heat value is available for these hydrocarbons near the critical point, a minimum value of 115 kJ/kg (50 Btu/lb) is sometimes acceptable as an approximation.

- PSV sizing for unwetted gas.

Relieving Temperature Calculation

From Hysys software (more accurate)

- Define one stream in Hysys 2. In this stream Temperature= normal operating temperature /Pressure= normal operating pressure /Vessel Composition Set molar flow to arbitrary value (e.g. 100 kmol/h)

- Read and copy actual volume flow from properties page

- Change operating pressure to relieving pressure

- Define Adjust, set stream temperature as adjusted variable

- Set actual volume flow as target object and define calculated value from step 3

- Run Adjust to calculate relieving temperature.

The recommended maximum vessel wall temperature, Tw, for the usual carbon steel plate materials is 1100 °F (593°C)

If F ' ≥ 182 in SI units (F ' ≥ 0.01 in USC units):

The minimum relief load recommended for sizing where F ' < 182 in SI units

(F ' < 0.01 in USC units) is calculated by setting F ' = 182 in SI units (F ' =

  - in USC units).

C13 is a constant [= 182 in SI units (0.01 in USC units)

To derive above equations, Z, Kb, and Kc in API 520-1:2014 have each

been assumed to have a value of 1.

Exposed Area:

1. Horizontal Drums

Up to 25 ft (7.6 m) above grade -> Use total exposed surface.

Greater than 25 ft (7.6 m) above grade -> Use the horizontal projected area of the

vessel

2. Vertical Vessels

The exposed surface within 25 ft (7.6 m) of grade is used. If the entire vessel

is more than 25 ft (7.6 m) above grade, then only the surface of the bottom

head need be included.

Thermal (Hydraulic) Expansion

The basic equation for calculating the pressure increase due to thermal

expansion in a piping system is as follows:

Typical pressure change versus temperature change for an 8 in. CS line without leakage are offered as follows:

the most common of which are the following:

 a) Piping or vessels are blocked in while they are filled with cold

liquid and are subsequently heated by heat tracing, coils,

ambient heat gain or fire.

 b) an exchanger is blocked in on the cold side with flow in the hot

side.

 c) Piping or vessels are blocked in while they are filled with liquid

at near-ambient temperatures and are heated by direct solar

radiation.

Since every application is for a relieving liquid, the required relieving rate is

small; specifying an oversized device is, therefore, reasonable.

 A DN 20 x DN 25 (NPS ¾” x NPS 1”) relief valve is commonly used

Two general applications for which thermal relieving devices larger

than a 3/4-inch x 1-inch nominal pipe size (NPS 3/4” x NPS 1) valve

might be required are long pipelines of large diameter in un-insulated

aboveground installations and large vessels or exchangers operating

liquid full.

A pressure-relief device might not be required to protect piping from

thermal expansion if :

a) the piping always contains a pocket of non-condensing vapor.

b) the piping is in continuous use and drained after being blocked-in using

well supervised procedures or permits;

c) the fluid temperature is greater than the maximum temperature expected

from solar heating [usually approximately 60 °C to 70 °C]

d) the estimated pressure rise from thermal expansion is within the design

limits of the equipment or piping.

e) Short sections of piping less than 100 ft (30 m) in length but not

exceeding 250 gallons (900 lit) in volume which can be blocked in,

generally do not need thermal relief valves. 88

For liquid-full systems, expansion rates for the sizing of relief devices

that protect against thermal expansion of the trapped liquids can be

approximated using below, in SI units:

If the duty of the heat exchanger is high enough to vaporize the liquid then use the equation bellow:

W= (T1-Tbp/T1-Tav)(Q/l)

L=latent heat of vap at relieving condition

Q=normal exchanger duty

T1=hot side inlet temperature

Tbp= cold side boiling temperature at relieving pressure

Tav= average of inlet and outlet temperature of cold side during normal operation.

Inadvertent Control Valve opening

The scenario to consider is that one inlet valve is in a fully opened position

regardless of the control-valve failure position. Opening of this control valve

can be caused by instrument failure or mis operation.

The required relieving rate is the difference between the maximum expected inlet flow and the normal outlet flow, adjusted for relieving conditions and considering unit turndown.   Or Subtract turn-down flow from calculated flow based on API-521 .

An important consideration is the effect of having a manual bypass on the inlet control valve(s) at least partially open.

If, during operation, the bypass valve is opened to provide additional flow, then this total flow (control valve wide open and bypass valve normal

position) shall be considered in the relieving scenario.

If the bypass is used only during maintenance to permit the control valve to be blocked in and removed from service, then the maximum flow of either the control valve or bypass valve needs to be considered.

If dp between design pressure of inlet and outlet of the valve is more than 1.3 hydrostatic pressure of outlet then bypass should be considered in psv sizing.

Gas breakthrough across a control valve can result in slug-flow high liquid

Velocities.

Relieving load calculation procedure

- calculate p1 = upstream design pressure and p2= downstream relieving pressure and by using fisher software calculate new cv and then use the equation in fisher handbook.

- Calculi the flowrate and then subtract turn-down flow from calculated flow.

Notes:

Gas-blow by

Sazeh heat exchanger

Column Relief Scenarios

Power Failure

Loss of Reflux

Loss of Feed

Loss of Condenser

Abnormal Heat Input by Reboiler

Vapor Blocked Outlet

Loss of Reflux

In order to maintain product separation the duty of the reboiler would be driven towards amaximum(Qreb_max).

Maximum Rebolier Duty= Uc x A x LMTD

Loss of Feed

Act accordingly and put an adjuster to equalize the volumetric flow of vapor to that of vapor from condenser and then subtract normal vapor from condenser from that. To do than put an adjuster and then assume molar flow as variable and volumetric flow as target and do the rest.

Condenser Loss

Copy like reflux scenario but:

Required Relief Load = Vapor – Normal Vapor from Condenser

Vapor Blocked Outlet

Relief Load @ conditions of stream @Prel equivalent to volumetric flow of

stream 'Vapor'

Heat Exchanger Tube Rupture

Heat exchangers and similar vessels should be protected with a relieving device of sufficient capacity to avoid overpressure in case of an internal failure.

In practice, an internal failure can vary from a pinhole leak to a complete tube rupture. A simplifying assumption of two orifices may be used.

The tube rupture scenario can be mitigated by increasing the design pressure of the low-pressure exchanger side (including upstream and downstream systems), and/or assuring that an open flow path can pass thetube rupture flow without exceeding the stipulated pressure, and/or providing

pressure relief.

Example:

HP 27 / LP 13

27 * 10/13 = 21.7 if the design pressure of low pressure side is increased to 21.7 barg then there is no need to have psv.

It may be necessary to locate the relieving device to be located either directly on the exchanger or immediately adjacent on the connected piping. This is especially important if the low-pressure side of the exchanger is liquid-full.

The following equations can be used to (preliminary) calculate the flow from the high to the low pressure side

12

The liquid equation will be used in the following cases :

− The liquid flow stays liquid as it flows through the tube break :

P2 > PVL

PVL vapor pressure of the liquid (high pressure fluid)

− The liquid flashes as it flows through the tube break

P2 < PVL

Then

- if the upstream pressure P1 is significantly above the liquid vapor pressure (P1 > PVL), use PVL instead of P2 and calculate WL based on P1 - PVL.

b) If the liquid is at boiling conditions on the high pressure side (or close to these conditions), the liquid flow WL can be calculated based on :

The pressure differential P1 - P2 with :

P1 = PVL

P2 : higher pressure of :

- critical flow downstream pressure (Pc). Experience has shown that PC should be calculated as PC = 0.55 P1, with Pc and P1 in absolute pressure units here,

- Relieving pressure P2.

−The mixed phase density (at relieving or critical flow conditions) instead of the liquid density

Review of the temperature condition

(*) The high-pressure side temperature is used to determine: T, ρL, PVL

A reasonable value is to consider the average operating temperature of this high pressure side. Nevertheless if the range of temperature is very large, flow through tube break should be calculated both at inlet and at outlet conditions.

Review of the calculated flowrate through tube break

− If the calculated discharge exceeds the normal total flow in the high pressure side, the latter flow should be used, except when sufficient volume on the HP side may supply the calculated flowrate above the normal flowrate.

Selection of PSV type

These devices include spring-loaded and pilot-operated Pressure Relief Valves (PRVs), rupture disk devices, and other pressure relief devices.

Conventional PRVs

- A conventional PRV is a self-actuated spring-loaded PRV which is designed to open at a predetermined pressure and protect a vessel or system from excess pressure by removing or relieving fluid from that vessel or system.

- Under normal system operating conditions, the pressure at the inlet is below the set pressure and the disc is seated on the nozzle preventing flow through the nozzle.

- The operation of a conventional spring-loaded PRV is based on a force balance.

- The spring load is preset to equal the force exerted on the closed disc by the inlet fluid when the system pressure is at the set pressure of the valve.

- In vapor or gas service, the valve may “simmer” before it will “pop.”

Conventional PRVs – Vapor Service

Conventional PRVs – Effects of Backpressure

Balanced PRVs – Effects of Backpressure

Pilot-Operated PRVs

Conventional PRVs (Applications & Limitations)

- Conventional spring opposed pressure relief valves are used in virtually all relief services which discharge to atmosphere or to a constant pressure system.

- They should not be used in applications which have variable back pressures

- They should not be used in applications which built-up back pressures in excess of the valve’s tolerance.

- Wherever possible, they are preferred to Balanced type PSVs.

Balanced Bellows PRVs (Applications & Limitations)

- They can be of two main types: balanced piston and balanced bellows.

Balanced bellows shall be given preference where the fluid is corrosive or fouling.

- Application - Balanced bellows PR valves should be specified where any of the following apply:

  - Excessive fluctuation in superimposed back pressures.

  - The built up back pressure exceeds 10% of the set pressure, based on gauge pressure; or it exceeds 21% of set pressure in the case of fire.

  - The service is fouling or corrosive, since the bellows shields the spring from process fluid.

- Balanced bellows PR valves may be used satisfactorily in vapor and liquid service with a total back pressure (superimposed plus built-up) as high as 50% of set pressure.

Pilot-Operated PRVs (Applications & Limitations)

- They can be of two main types: piston or diaphragm. Safety-wise, none of these is given preference but only types with non-flowing pilots shall be used.

- The pilots may be either a flowing or non-flowing type. The flowing type allows process fluid to continuously flow through the pilot when the main valve is open; the non-flowing type does not. The non-flowing pilot type is generally recommended for most services to reduce the possibility of hydrate formation (icing) or solids in the lading fluid affecting the pilot’s performance.

- Advantages - The advantages of pilot-operated PR valves are as follows:

  - A pilot-operated valve is capable of operation at close to the set point and remains closed without simmer until the inlet pressure reaches above 98% of the set pressure.

  - It may be less subjective to chattering. A modulating pilot valve may also be considered where chattering is a potential problem.

  - Its opening pressure is unaffected by back pressure, and high built-up back pressure does not result in chattering. No decrease in capacity occurs as long as flow through the valve is critical.

  - Back pressure up to 75% of set pressure may be used, The valve manufacturer should be consulted on any application where the total back pressure may exceed 75% of set pressure.

- Disadvantages - Pilot-operated PR valves are subject to the following disadvantages:

- They are not recommended for dirty or fouling services, because of plugging of the pilot valve and small-bore pressure sensing lines.

- They are normally limited to a maximum inlet temperature of 450°F (280°C) by the “O" ring piston seals.

- In smaller sizes (below 6”), pilot operated PR valves are more costly than spring operated PR valves.

Piston for pressure vessel and diaphgram for tanks./ note that because of safety non-flowing type should be used.

Notice to pilot exhaust whether it is discharged to atmosphere or safe location . it creates some problems for you notice that.

For pilot mop/sp is less than 0.98 and for spring-loaded it is less than 0.9.

Chattering

- Chattering is the rapidly alternating opening and closing of a PR valve.

- This vibration may result in misalignment and leakage when the valve returns to its normal closed position. If chattering continues for a sufficient period, chattering may result in mechanical failure of valve internals or associated piping fittings.

- The principal identified causes of PR valve chattering are oversized valve, excessive inlet pressure drop, excessive built-up back pressure incorrect blow-down ring setting, and liquid surge.

Oversized Valve:

- Typically, a flow through the valve in vapor service equal to at least 25% of its rated capacity is necessary to keep the disc in the open position.

- Liquid service valves are less likely to chatter at low relieving rates, and they will modulate down to about 10% of rated capacity before chatter becomes a problem.

- ++++ oversize causes chaterring and choose modulate pilot

Excessive Inlet Pressure Drop:

- Experience as well as manufacturers' recommendations dictate an inlet pressure drop of no more than 3% of set pressure at the PR valve rated capacity.

Excessive Built-up Back Pressure:

- Built-up back pressure resulting from discharge flow through the outlet system of a conventional PR valve results in a force on the valve disc tending to return it to the closed position. If this returning force is sufficiently large, it may cause the valve to close, only to reopen immediately when the effect of built-up back pressure is removed.

Blowdown Ring Settings:

- In some cases, incorrect blowdown ring settings have resulted in valve chattering.

Multiple Pressure Relief Valve

- In certain cases, it is necessary to install two or more PR valves in parallel for a single service.

- Large Release: The magnitudes of some large releases may be greater than the capacity of the largest single PR valve that is commercially available at the desired pressure rating, necessitating the use of two or more valves. For instance we do not have orifice area larger than 26inch.

- Preventing Chattering: Where different contingencies of equal probability require substantially different capacities, it is always best to use two or more PR valves with staggered settings. The lower capacity valve in this case would be at the lower staggered set pressure. For instance, there is one case of 100000 and 1000. We size for both.

- Preventing Chattering: When a fire contingency is the largest contingency and the next largest contingency is less than 25% of the fire relieving rate, multiple PR valves with staggered settings should always be used. However, when the fire contingency is the smallest load, it is generally ignored. Example that 100000 fire case and 1000 non-fire case we size for both but if 100000 non-fire and 1000 fire then we size just for non-fire because fire is a remote case.

Sizing Procedure

/PRVs may be initially sized using the equations presented API-520 as

appropriate for vapors, gases, liquids, or two phase fluids.

/The designer can use API 526 to select a PRV. API 526 is a purchase

specification for steel flanged valves

/These equations utilize effective coefficients of discharge and effective

areas which are independent of any specific valve design. In this way, the

designer can determine a preliminary PRV size.

/The rated coefficient of discharge for a PRV, as determined per the

applicable certification standards, is generally less than the effective

coefficient of discharge used in API 520.

/For this reason, the actual discharge or orifice area for most valve designs is

greater than the effective discharge area specified for that valve size per API

526.

Sizing Procedure – Gas or Vapor Relief

The sizing equations for pressure relief devices in vapor or gas service

assume the pressure-specific volume relationship along an isentropic path.

Critical Pressure:

/If the pressure downstream of the nozzle is less than, or equal to, the critical

flow pressure, then critical flow will occur.

/If downstream pressure exceeds the critical flow pressure, then subcritical

flow will occur.

Sizing Procedure – Gas or Vapor Relief – Critical flow

Sizing Procedure – Gas or Vapor Relief – Sub Critical flow

When the ratio of backpressure to inlet pressure exceeds the critical

pressure ratio Pcf /P1, the flow through the pressure relief device is

subcritical. For Conventional and Pilot-operated PRVs below equation could

be used.

Sizing Procedure – Liquid Relief : PRVs Requiring Capacity Certification

The ASME Code requires that capacity certification be obtained for PRVs

designed for liquid service. The procedure for obtaining capacity certification

includes testing to determine the rated coefficient of discharge for the liquid

PRVs at 10 % overpressure.

After the Reynold’s Number, Re, is determined, the factor Kv is obtained from

above. Kv is then applied in PSV Area Calculation Equation to correct the

preliminary required discharge area. If the corrected area exceeds the chosen

standard orifice area, the above calculations should be repeated using the

next larger standard orifice size.

Rupture Disk

/Rupture disk devices are non-reclosing pressure relief devices .They are

used in single and multiple relief device installations.

/With no moving parts, rupture disks are simple, reliable, and faster acting

than other pressure relief devices.

/Advantages :

 No simmering or leakage prior to bursting.

 Rupture discs can open fully within 1 millisecond for vapor/gas systems,

thus making them more effective than PR valves when the overpressure

is caused by sudden pressurization (for example as a result of a tube

failure in a high pressure heat exchanger).

 Less expensive to provide corrosion resistance.

 Less tendency to foul or plug.

 Can provide both depressuring and overpressure protection.

 Lower initial cost than for an equivalent service PR valve.

/Disadvantages :

 Non-reclosing pressure relief device. Replacement of the burst rupture

disc is required to allow continued operation if it is the only protective

device.

 Require periodic replacement.

 Greater sensitivity to mechanical damage.

 Greater sensitivity to temperature.

/A stand-alone rupture disk is used when:

 You are looking for capital and maintenance savings

 You can afford to loose the system contents

 The system contents are relatively benign

 You need a pressure relief device that is fast acting

 A relief valve is not suitable due to the nature of the system contents

/When to use combination of RD and PSV?

 The system contains a toxic substance and you are concerned that the

relief valve may leak.

 The system contains solids that may plug the relief valve over time.

 If the system is a corrosive environment, the rupture disk is specified

with the more exotic and corrosion resistant material.

Rupture Disk (Sizing)

Resistance to Flow Method

 The Resistance to Flow Method analyzes the flow capacity of the relief

piping. The analysis takes into account frictional losses of the relief piping

and all piping components.

 Piping component losses may include nozzle entrances and exits, elbows,

tees, reducers, valves and the rupture dis

RD contribution to the overall frictional loss in the piping system is

accomplished by using “Kr”, which is analogous to the K value of other

piping components.

 Kr is determined experimentally in flow laboratories by the manufacturer for

their line of products.

 If at the time of sizing the manufacturer and model of the rupture disk are

unknown, API STD 520 recommends using a Kr of 1.5.

 ASME Section VIII, Division 1 states that a Kr of 2.4 shall be used.

Type of Rupture Disks

This rupture disk typically has an angular seat design and provides a

satisfactory service life when operating pressures are up to 70 % of the

marked burst pressure of the disk (70 % operating ratio).

These disks have a random opening

pattern and are considered fragmenting

designs that are not suitable for

installation upstream of a PRV.

Some designs provide satisfactory service life when operating pressures are

up to 85 % to 90 % of the marked burst pressure of the disk (85 % to 90 %

operating ratio).

Because the score lines control the opening

pattern, this type of disk can be

manufactured to be non-fragmenting and

is acceptable for installation upstream of a

PRV.

Type of Rupture Disks :

Reverse-acting rupture disks may be manufactured as non-fragmenting and

are suitable for installation upstream of PRVs. These disks provide

satisfactory service life when operating pressures

are 90 % or less of marked burst pressure (90 %

operating ratio).

Because a reverse-acting rupture disk is operated

with pressure applied on the convex side, thicker

disk materials may be used, thereby lessening the

effects of corrosion, eliminating the need for

vacuum support, and providing longer service

life under pressure/vacuum cycling conditions

and pressure fluctuations.

211