USV-2071

Sizing and Design Principle

1.Operating Condition

…………………………………………………………………………………………………………….

2. Body/Bonnet

In Body/Bonnet Tab the following information should be determined:

1.Type

2. Size

3.Rating

4.Connection type

5.Body material

6.Pipe inlet and outlet

Type:

Since the valve is used as tight shut-off valve, based on above explanation, butterfly or ball

valve should be used and due to the fact that a line-size body and a full-bore trim shall be used,

among these choices ball type is selected since the line sizing is 3 in. which is less than 4

in.

Rating:

The rating of valves is the same as the rating of connecting pipes. Since the adjacent pipe class

is 600, the valve class would be 600.

Connection type:

The three most common methods of installing control valves into pipelines are by means of

screwed pipe threads, bolted gasketed flanges, and welded end connections.

Screwed end connections, popular in small control valves, offer more economy than flanged

ends. The threads usually specified are tapered female NPT (National Pipe Thread) on the

valve body. They form a metal-to-metal seal by wedging over the mating male threads on the

pipeline ends.

This connection style, usually limited to valves NPS 2 (DN 50) or smaller, is not recommended

for elevated temperature service. Valve maintenance might be complicated by screwed end

connections if it is necessary to take the body out of the pipeline because the valve cannot be

removed without breaking a flanged joint or union connection to permit unscrewing the

valve body from the pipeline.

Flanged end valves are easily removed from the piping and are suitable for use through the

range of working pressures for which most control valves are manufactured. Flanged end

connections can be used in a temperature range from near absolute zero to approximately

815°C (1500°F). They are used on all valve sizes. The most common flanged end connections

include flat-face, raised-face, and ring-type joint.

| Screwed End | Flanged End | Welded End |
| --- | --- | --- |
| 2” and smaller | Up to class 900 | Suitable for class 1500 and 2500 |

Since the valve type is ball , flange type with RF  is used for connection of the valve to

adjacent piping,

Body material

1.A216 WCB/WCC or forged carbon steel, A105 is used in non-corrosive services from -28 to

427C.

2. If there are some severe conditions such as flashing, it is typical to use A217 WC9

3. For high temperature services like steam let-down station or HHPS it is a practice to use

A217 WC6.

4. A351 CF8 is used mostly for combined flashing and corrosive services and for temperatures

below -28C.

5. For oxygen services, it is highly recommended to use Monel.

Based on the next-page Table, A 216 WCB is selected

Body Size

Body size = Line size = 3 inch

Valve Body Material Selection based on Fluids

| Fluid | Material |
| --- | --- |
| NG | SA216 WCB |
| Purge Gas | SA216 WCB |
| Syngas | SA216 WCB/A351 CF8 |
| Process Condensate | A351 CF8 |
| LPS | SA216 WCB |
| MPS | SA216 WCB |
| HPS | SA216 WCB /SA217 WC6 |
| HHPS | SA217 WC6 |
| LPC | SA217 WC6/WC9 |
| MPC | SA216 WCB |
| HPC | SA217 WC9 |
| BFW | SA216 WCB/ SA217 WC6 |
| WMW | A351 CF8 |
| Nitrogen | SA216 WCB |
| Crude Methanol | SA216 WCB |
| Flashed Methanol | A351 CF8 |
| Refined Methanol | SA216 WCB |
| Process Gas | SA216 WCB/ A351 CF8 |
| Oxygen | Monel |

3. Trim

For Trim Tab the following should be specified:

1.Cv required

2.Charachteristic

3.Type

4.Material

5.Leakage Class

Cv calculation

Cv required = line sized

Trim type

Full-bore is selected when a ball valve is selected.

Trim Charachteristic

ON/OFF

Trim material

Based on above explenation AISI 316 is selected.

Leakage Class

Control valves are designed to throttle, but they are also often expected to provide some type of

shut-off capability.

A control valve's ability to shut off has to do with many factors: Balanced or unbalanced plug,

seat material, actuator thrust, pressure drop, and the type of fluid can all play a part in how wella

particular control valve shuts off.

Class IV is also known as a “Metal-to-Metal” seat classification. It is the kind of leakage rate you

can expect from a valve with a metal plug and metal seat

Class VI is known as a “Soft Seat” classification. Soft Seat Valves are those where either the plug

or seat or both are made from some kind of composition material such as Nitrile or Polyurethane.

For tight shut-off valves minimum V shall be selected

Actuators

for actuator and positioners Tab the following should be specified:

1.Type

2.Modulating or ON/OFF

3.Failiure position

4.dP for sizing

Actuator Type:

Based on above criteria, Piston with spring return is shall be selected

ON/OFF type is selected.

Failure position

Control valves shall be such that on air failure the valve takes automatically a safe position

either open, or close, or locked in position, depending upon the process requirements.

Based on process requirement FC is selected.

dp for sizing

dp equals design pressure which is 52 barg

Solenoid Valve

Type = 3 way

Hand wheel

Generally, no hand wheel for TSOV is used.