T-5004

Packed Tower Design and Principles

Packed Tower Data Input

Vapor inlet                                                                 Vapor Outlet

Liquid Inlet                                                                Liquid Outlet

Packed Tower ID Sizing

1.Determine the capacity term value

Choose a ΔP value and calculate the flow parameter

Calculation for this step

| Inlet | Value |
| --- | --- |
| L | 8197 |
| G | 11332 |
| Lm | 2.2769 |
| Gm | 3.14 |
| F | 0.024 |

| Outlet | Value |
| --- | --- |
| L | 8524 |
| G | 11004 |
| Lm | 2.36 |
| Gm | 3.05 |
| F | 0.026 |

2. Determine Capacity term or C from diagram below

Packed Column Pressure Drop Correlation

Calculation for this step

Note that according to licensor requirement a dp of 10 mbar is chosen

3.

3. Choose a packing type and determine the packing factor (Fp)

- The packing factors for various packings are shown in figure 18.

- Usually packings smaller than 25 mm (1 in) size are intended for column diameters of 300 mm or

smaller, packings of 25 mm to 37 mm (1 in to 1½ in) in size for column diameters from 300

mm to 900 mm, and packings from 50 mm to 75 mm (2 in to 3 in) in size for column diameters

of 900 mm and more.

Calculation for this step

We select IMTP because of its  wide applicability and suppose that ID is more than 900mm; as

a result, a 2-inch (50mm) IMPT is selected with packing factor of 18 according to the table

below.

4. Calculate the gas (or vapour) mass velocity G with the capacity term C value determined in

previous step.

5. Determine minimum inside diameter using following formula

Calculation for this step

| Inlet Parameter | Value |
| --- | --- |
| ρl | 989 kg/m3 |
| ρv | 1.15 kg/m3 |
| C | 0.0175 |
| Fp | 18 |
| G^2 | 3.49 |
| G | 1.86 |
| D | 1464 mm |

| Outlet Parameter | Value |
| --- | --- |
| ρl | 964kg/m3 |
| ρv | 1.11 kg/m3 |
| C | 0.0175 |
| Fp | 18 |
| G^2 | 3.27 |
| G | 1.81 |
| D | 1466 mm |

6. KG TOWER Confirmation

Open the software and select a new case

Fill-up box 1 and 2 and then click Packing

Fill-up the box accordingly and Click Options and select Constant liquid

Comparison

| Haldor Topsoe | Excel Sheet | KG Tower |
| --- | --- | --- |
| 1500 | 1465 | 1250 |

Notice that if the height of the tower is 5 m then the total dp according to KG Tower is 10 mbar it means that the minimum ID for the packed tower is 1250.